CHE 4151										          Fall, 2016

Chemical Reactor Design									Jennings

HOMEWORK  # 10  ─  Nonisothermal  Gas-Phase  MFRs

( due  Friday,  11 / 4 )

1.	[Problem 5-29, pp.245-246 in Schmidt.]  A catalyst has been developed for the oxidative dehydrogenation of ethane to ethylene:

2 C2H6   +   O2    →    2 C2H4   +   2 H2O

The heat of reaction is −25 kcal/mol.A.  The process produces only ethylene when carried out in an adiabatic mixed-flow reactor at 2 atm pressure.  The feed contains 5% ethane, 19% oxygen, and 76% nitrogen at 300 K.  The heat of reaction is –25 kcal/mol.ethane.  Assume that the heat capacity of all components is 7 cal/mol-K.  If the reaction goes to completion:  (a) calculate the moles of gas produced for each mole fed to the reactor, (b) calculate a value of the effluent temperature, and (c) calculate a value of the ratio of space time to residence time.

Solution:

(a)				2 C2H6 + O2 --> 2 C2H4 + 2 H2O

C2H6 used 				=  1 kmol

O2 required				= 1 kmol

O2 supplied				= 0.19 kmol (19%)

N2 supplied along with O2		= 0.19*(76/21) = 0.6876

Air supplied  = O2 supplied + N2 supplied = 0.19 + 0.6876 = 0.8776 kmol

Weight of air supplied = 0.8776*29 = 25.45 kg

C2H6 taken = 1 kmol = 1*30 = 30 kg

Weight ratio of air to C2H6 in feed = (25.45/30) = 0.848

Gases leaving:

H2O, C2H4, O2, C2H6

C2H6 reacted = 1 kmol

O2 leaving = O2 supplied - O2consumed =  0.19 - (0.19/2) = 0.095 kmol

N2 leaving = N2 entering = 0.6876 kmol

C2H4 leaving = C2H4 produced = 1 kmol

H2O leaving = H2O produced =  1 kmol

(b)  The formula for computation is,

T=To+(-∆HRx)X∑ΘiCpi

by substituting the known values we get,

T=300+--25000*0.05∑0.05*7+0.19*7+0.76*7

T=300+178.57=478.57 K

(c) From Fogler pg.no-880, by derivation for the given  condition  is found to be 1:1, where residence time equals to space time.

2.	[Problem 4-2, p.209  in Hayes.]  A mixed-flow reactor is to be designed to produce butadiene from butene by dehydrogenation (gas-phase reaction), C4H8   →   C4H6  +  H2.  The

first-order reaction rate expression is written in terms of the partial pressure of butene as:

rC4H8   =   k pC4H8mol/L-hr

The rate constant (k) may be expressed as

k  =  (1.666 x 1015) e−30,109 / Tatm–1-L–1-hr–1

The reaction is endothermic with a heat of reaction of110 kJ/mol.butene.  The reactor is often operated adiabatically, therefore steam is often added to the feed to provide thermal energy for the reaction.  For this reactor, the feed is a mixture of 10 mol of steam for each mol of butene.  The reactor pressure is 2 atm and the feed temperature is 650oC.  The heat capacity of the

feed stream may be considered constant at 2.1 kJ/kg-K.

a)	Calculate the reactor volume for a 20% conversion of butene if the reactor is operated isothermally at 650oC with a total inlet molar flow rate of 11,000 mol/hr.

b)	Determine the reactor volume for 20% conversion of butene for a total inlet molar feed rate of 11,000 mol/hr in an adiabatic reactor.

Solution:

(a)  The formula for computing reactor volume is,

V=FAOX(1-0.5X)2kCAO2(1-X)2

by substituting the given values for part A and before that temperature of 650oC (923.15K) is substituted in equation k for computation.

k=1.666*1015e-30,109T=1.666*1015e-30,109923.15=11.4

so we get,

V=11000*0.2*(1-0.5(0.2))211.4*102*(1-0.2)2=2.442 L

(b) For adiabatic reactor, the formula to compute the volume is,

Q=mCp∆T=-∆H*k*V

V=mCp∆T-∆H*k

by substituting the given values we get,

V=10*2.1*103*25-11000*11.4=4.18 L

3.  [Problem 8-6, p.298  in Roberts.]  Methyl cyclohexane (M) is being dehydrogenated to toluene (T) [ M → T + 3 H2 ] in a catalytic,  mixed-flow reactor.  The feed to the reactor is a 2:1 (molar) mixture of hydrogen and methyl cyclohexane at a temperature of 500oC and atmospheric pressure.  The feed rate of methyl cyclohexane is 1.2 mol/s.  The reactor contains 10.0 kg of catalyst and operates adiabatically.  At what temperature and fractional conversion of methyl cyclohexane does the reactor operate?

Additional data:

ΔĤR0  =  54.3 kcal/mol.M

ĉP,M = 32.3 cal/mol-K    ,    ĉP,T = 24.3 cal/mol-K    ,    ĉP,H2 = 6.89 cal/mol-K

k = (4.31x1033) e–35,000 / T L/kg.cat-s

Solution:

The formula for temperature computation is,

T=To+(-∆HRx)X∑ΘiCpi

by substituting given values we get,

500+273.15K=T+-54300*X10*(32.3+24.3+6.89)

T=773.15+85.52X

The temperature and conversion are correlated  and can be computed by finding any one of the unknown parameters,

for k      		k=4.31*1033e-35000T      so @ 500oC (773.15 K) the value of k,

k=4.31*1033e-35000773.15=9.42*1013 Lkg.cat-s

The conversion is computed by using,

X=∑CPAT-TO-∆HRx= -32.3+24.3+6.89*773.15-298.15-54.3*103

X=0.55

By substituting the conversion factor of X=0.55 in the above equation we get temperature as,

T=773.15+85.520.55

T=820.19 K or547.036oC

4.  [Problem 8-7, pp.298-299  in Roberts.]  Methyl cyclohexane (M) is being dehydrogenated to toluene (T) [ M → T + 3 H2 ] in a catalytic,  mixed-flow reactor.  The feed to the reactor is a 2:1 (molar) mixture of hydrogen and methyl cyclohexane at a temperature of 500oC and atmospheric pressure.  The feed rate of methyl cyclohexane is 1.2 mol/s.  The reactor contains 10.0 kg of catalyst and operates adiabatically.  There is a heating coil in the reactor with hot flue gases flowing through the coil at a temperature of 600oC.  The overall heat transfer coefficient between the coil and the reactor contents is 20.0 cal/m2-K-s.  If the desired fractional conversion is 0.50, at what temperature should the reactor operate?  What area of heating coil is necessary to maintain that temperature?

Additional data:

ΔĤR0  =  54.3 kcal/mol.M

ĉP,M = 32.3 cal/mol-K    ,    ĉP,T = 24.3 cal/mol-K    ,    ĉP,H2 = 6.89 cal/mol-K

k = (4.31x1033) e–35,000 / T L/kg.cat-s

Solution:

As heater is attached with reactor the formula for computing the data's for conversion rate is,

X=(UAFAoT-Ta)+∑θiCPiT-To-∆HRx

by computing the given data's we get,

X=20*A1.2*T-873.15+(634.9*T-773.15)-54300

for solving fractional conversion X=0.50

0.50=(16.67*A*T-873.15+(634.9*T-773.15)-54300

-27150=(16.67*A*T-873.15+(634.9*T-773.15)

from previous problem it is found T = 820.19 K for conversion @ X=0.55 so by approximation we get A,

-27150=(16.67*A*820.19-873.15+(634.9*(820.19-773.15)

-27150-29865=-882.8432*A

A=-57015-882.8432

A=64.58 m2

Example Problems

( no solution needed )

[Problem 8-2, p.296  in Roberts.]  The elementary gas-phase isomerization reaction,  A  ↔  B ,  is being carried out in a fluidized-bed reactor (which may be modeled as an ideal mixed-flow reactor) containing 10 kg of catalyst.  The adiabatic operating conditions are: temperature = 300oC and pressure = 1 atm.  The feed is 40% A and 60% inert carrier gas (C) at a total volumetric flow rate of 50 m3/hr (at STP).

a)	Under these conditions, what will be the fractional conversion of A?

b)	What feed temperature is required to operate at steady-state at these conditions?

Additional information:

kf   =   14.9 L/g.cat-hr  at  300oC

KEQ  =  4.19 at 300oC

ΔĤR,A =  + 3.50 kcal/mol.A

ĉP,A  =  ĉP,B  =  17.5 cal/mol-K   ,   ĉP,C  =  7.0 cal/mol-K

[Problem 8-E. (original)]The elementary, irreversible, gas-phase reaction, A + B → D, is to be carried out in a 10-liter mixed-flow reactor.  The feed contains only A and B in stoichiometric proportions at 580 kPa and 77oC.  The molar feed rate of A is 20 mol/s.  Calculate the amount of heat that must be added to or removed from the reactor to maintain an operating temperature of 500 K.

Additional  Data:

Rate law parameters:

k    =  0.065  L/mol-min  at  25oC

E*  =  70  kJ/mol

Thermodynamic parameters at 25oC:

ĤF,A  =  –40 kJ/mol     ,     ĉP,A  =  25 J/mol-K

ĤF,B  =  –30 kJ/mol     ,     ĉP,B  =  15 J/mol-K

ĤF,D  =  –45 kJ/mol     ,     ĉP,D  =  35 J/mol-K