1. C&A makes two types of products using four machines from 9 a.m. to 5 p.m. each day. Product A visits machines 1, 2, and 4. Product B only visits machines 1 and 3. The capacity is 0.4 unit per minute at machine 1, 0.12 unit per minute at machine 2, 0.2 unit per minute at machine 3, and 0.3 unit per minute at machine 4. The demand per day is 40 units for Product A and 160 units for Product B. What is the implied utilization for machine 1?

a. 1.67

b. 1.04

c.  0.69

d. 0.28

Answer:

B. 1.04

Explanation:

Total demand rate for machine 1 = 40 + 160 = 200 units per day.

Capacity for machine 1 = 0.4 unit per minute or 0.4 × 8 × 60 units per day.

Implied utilization for machine 1 = 200/ (0.4 × 8 × 60) = 1.04.

2. C&A makes two types of products using four machines from 9 a.m. to 5 p.m. each day. Product A visits machines 1, 2, and 4. Product B only visits machines 1 and 3. The capacity is 0.4 unit per minute at machine 1, 0.12 unit per minute at machine 2, 0.2 unit per minute at machine 3, and 0.3 unit per minute at machine 4. The demand per day is 40 units for Product A and 160 units for Product B. Which resource is the bottleneck?

Machine 1

Machine 2

Machine 3

Machine 4

Answer: Machine 3

Explanation:

Total demand rate for machine 1 = 40 + 160 = 200 units per day.

Total demand rate for machine 2 = 40 units per day.

Total demand rate for machine 3 = 160 units per day.

Total demand rate for machine 4 = 40 units per day.

Implied utilization for machine 1 = 200/ (0.4 × 8 × 60) = 1.04.

Implied utilization for machine 2 = 40/ (0.12 × 8 × 60) = 0.69.

Implied utilization for machine 3 = 160/ (0.2 × 8 × 60) = 1.67.

Implied utilization for machine 4 = 40/ (0.3 × 8 × 60) = 0.28.

The bottleneck is the resource with the highest implied utilization, which is machine 3.

3. C&A makes two types of products using four machines from 9 a.m. to 5 p.m. each day. Product A visits machines 1, 2, and 4. Product B only visits machines 1 and 3. The capacity is 0.4 unit per minute at machine 1,

0.12 unit per minute at machine 2, 0.2 unit per minute at machine 3, and

unit per minute at machine 4. The demand per day is 40 units for Product A and 160 units for Product B. Assume the relative proportion between the two product types remains constant. What is the flow rate for Product B in units per day?

24

96

C. 120

D. Cannot be determined

Answer: 96

Explanation:

Because the implied utilization is 1.67 (exceeds 1), the process is capacity constrained.

Flow rate for Product B = Demand rate/Implied utilization of the bottleneck =

160/1.67 = 96 units per day.

4. About 80% of customers who receive a promotion coupon will visit the store, but only 20% of those in the store will make a purchase. What is the yield of the promotion?

a. 80%

b.60%

c. 20%

d. 16%

Answer: 16%

Explanation:

Process yield = 0.8 × 0.2 = 0.16.

5. About 80% of customers who receive a promotion coupon will visit the store, but only 20% of those in the store will make a purchase. How many promotion coupons must be sent to get 100 customers to make a purchase?

a. 1600

b. 625

c. 500

d. 125

Answer: 625

Explanation:

Process yield = Number of promotion coupons = 100/0.16 = 625.

Number of coupons started to get Q coupons = Q/Process yield = 100/0.16

= 625.