# Florida Institute of Technology

# BUSINESS FTMT011B

# QUIZ 3

# 1)Which of the following is the cost of quality classification for costs such as cost incurred when the firm assesses the performance level of its process?

  - Appraisal costs

  - Prevention costs

  - External failure costs

  - Customer return cost

  - Workmanship costs

Answer:  A   Slides_Session 9, page 6.

# A point which is outside of the lower control limit on a p-­‐chart

  - is an indication that no cause of variation is present

  - should be ignored because it signifies better than average quality

  - should be investigated because an assignable cause of variation might be present

  - should be ignored unless another point is outside that limit

  - is impossible since the lower limit is always zero Answer:  C	Slides_Session 10, page 6.

# You have just used the capability index (Cpk) formulas to compute the two values "min [1.2, 1]". Which of the following is the interpretation of these numbers?

  - The true capability index value is 1.

  - The mean of the production process has shifted to the left of the design limits.

  - The mean of the production process has shifted to the right of the design limits.

  - The mean has not shifted at all.

  - The true capability index value is between 1.2 and 1. Answer:  C	Slides_Session 10, page 39.

# Lean production

  - works in all circumstances.

  - never runs out of inventory.

  - requires a stable environment

  - may run underproduction.

  - requires preventive maintenance.

Answer:	C	Slides_Session 12, page 5.

# In setting up a kanban control system you need to determine the number of kanban card sets needed. If the expected demand during lead time is 25 per hour, the safety stock is 20% of the demand during lead time, the container size is 5, and the lead time to replenish an order is 5 hours, what is the number of kanban card sets needed?

  - 5

  - 20

  - 27

  - 30

  - 34

Answer:  D       k=(25)(5)(1.2)/5=30, Slides_Session 12, page 17.

# A company wants to forecast demand using the weighted moving average. If the company uses four prior yearly sales values (i.e., year 2003 = 100, year 2004

= 120, year 2005 = 140, and year 2006 = 210), which of the following is the weighted moving average forecast with w1=0.4, w2=0.3, w3=0.2, w4=0.1 for year 2007?

A. 100

B. 125

C. 145

D. 155

E. 160

Answer:  E	Slides_Session 13, page 18.

0.4(210)+0.3(140)+0.2(120)+0.1(100)=84+42+24+10=160.

# A company wants to forecast demand using the Simple Exponential Smoothing. If the company uses four prior yearly sales values (i.e., year 2003 = 100, year 2004 = 120, year 2005 = 140, and year 2006 = 210), which of the following is the Exponential Smoothing

forecast with	=0.5 for year 2007?

| A. 140 | Year | At | Ft |
| --- | --- | --- | --- |
| B. 155.5 | 2011 | 100 | 100 |
| C. 160 | 2012 | 120 | 100=0.5(100) +(1-­‐0.5) (100) |
| D. 167.5 | 2013 | 140 | 110=0.5(120) +(1-­‐0.5) (100) |
| E. 170.5 | 2014 | 210 | 125=0.5(140) +(1-­‐0.5) (110) |
|  | 2015 | ? | 167.5=0.5(210) +(1-­‐0.5) (125) |

Answer:  D	Slides_Session 13, page 20.

Exhibit 3-­‐1

The demand for the first 3 periods and the Simple Exponential Smoothing forecast with =0.5 for the first four periods are as follows:

| Period | Demand | Forecast | Forecast Error | Forecast Error |
| --- | --- | --- | --- | --- |
| 1 | 23 | 23 |  |  |
| 2 | 22 | 23 | -­‐1.0 | 1 |
| 3 | 24 | 22.5 | 1.5 | 1.5 |
| 4 | ? | 23.25 | 0.25=[(-­‐1.0)+1.5]/2 | 1.25=(1.0+1.5)/2 |
|  |  |  | MFE | MAD |

# Based on Exhibit 3-­‐1, calculate MFE an MAD

  - MFE=1.6667 and MAD=0.8333 (If you count first period forecast error zero).

  - MFE=0.25 and MAD=1.25

  - MFE=1.25 and MAD=1.50

  - MFE=1.20 and MAD=1.45

  - MEF=0 and MAD=1.25 Answer:  B

# In business forecasting, what is usually considered a short-­‐term time period?

- Four weeks or less

- More than three months

- Six months or more

- Less than three months

- One year

Answer:  D	Slides_Session 14, page 18.

# From Sport Obermeyer case, we learnt that risk-­‐based production sequencing strategy is

- a strategy that uses reactive production capacity to make low-­‐risk products and postpones production of high-­‐risk products until after additional market information reduces those products’ demand uncertainty.

- a strategy that uses non-­‐reactive production capacity to make low-­‐risk products and postpones production of high-­‐risk products until after additional market information reduces those products’ demand uncertainty.

- a strategy that uses non-­‐reactive production capacity to make high-­‐risk products and postpones production of low-­‐risk products until after additional market information becomes available.

- a strategy that uses reactive production capacity to make high-­‐risk products and postpones production of low-­‐risk products until after additional market information becomes available.

- None of above

Answer:  B	Slides_Session 15, page 27.