Louisiana State University

CM 3201

1)One ton of air conditioning is equivalent to:

  - 12,000 btuh

- Air-cooled equipment for heat rejection handle much smaller tons of capacity when compared to the tons of capacity for water-cooled heat rejection equipment

  - True

- Since heat pumps can be used for heating and cooling purposes, typically one will see heat pumps with a SEER and a HSPF value.

  - True

- A fuel oil	is a boiler component that provides atomized fuel oil to the boiler.

  - Burner

- A	draft is a mechanical draft created by air pushed through the boiler firebox by a blower or fan located at the front of a boiler.

  - Forced

- A hydronic heating system uses a boiler to heat water that is then transported through supply lines to spaces within a facility.

  - True

- The flue gas, which is a by-product of the combustion occurring in boilers and furnaces, is non- toxic and may or may not be exhausted to the outdoors.

  - False

- The vapor compression refrigeration cycle uses an absorbent such as lithium bromide

  - False

- A heating plant for a large medical campus requires 22,000 MBH (1MBH = 1,000,000 btuh) of heating capacity. Which of the following pipe size and velocity in feet per second (fps) for the main would be needed if the plant is a 50 psig medium-pressure steam system?

  - 8" pipe at 130 fps

- Which of the following HVAC split systems has ductwork found above the furnace and cooling coil?

  - Up-flow

- Which of the following would represent a common coefficient of performance value for an A/C system that uses the vapor compression cycle?

  - None of these

- A split system for air conditioning separates the evaporator from the condensing unit.

  - True

- An oversized cooling system will not only cost more, but can result in short cycling of the system

  - True

- When a heat pump is in the cooling mode, the	unit is the evaporator and the	unit is the condenser.

  - : indoor; outdoor

- What does the compressor do in the vapor compression refrigeration system?

  - The compressor takes the low-pressure refrigerant vapor from the evaporator and compresses it to a higher pressure

- The flue gas, which is a by-product of the combustion occurring in boilers and furnaces, is non- toxic and may or may not be exhausted to the outdoors.

  - False

- A hydronic heating system uses a boiler to heat water that is then transported through supply lines to spaces within a facility.

  - True

- The refrigeration cycle uses a negative expansion valve to obtain either heating or cooling from a heat pump

  - False

- Which type of fan is used for air handling and return air?

  - Centrifugal fan

- Minute pressure changes can be expressed in	of water column.

- Inches

  - Width

- Single zone VAV systems are more energy efficient than CV terminal reheat systems

  - True

- The dust spot efficiency test used for rating and testing air filters uses atmospheric dust that is passed through the filter and the discoloration of the dust passing through the filter is observed.

  - True

- This phenomenon occurs when an air stream of conditioned air moves through a space

  - Coanda Effect

- A constant volume HVAC system uses which of the following principles to provide comfort to a space?

  - Varying the temperature of the air supplied while holding a constant flow of air

- In a multiple-zone VAV system, a master thermostat is used to control the temperature of the entire building.

  - False

- A reverse return HVAC piping system provides the first unit in the system with the longest supply and the shortest return piping

  - True

- Water pressure can be expressed in feet of head or in pounds per square inch

  - True

- When sizing rectangular ductwork, which of the following dimensions should be stated first?

  - Width

- Registers are composed of two components - Dampers & grilles.

  - True

- A duct is sized to allow the required amount of air for proper heating or cooling to flow to each building space.

  - True