# St. Petersburg College

# FFP 1111

# Chapter 7 Quiz

1)Which two gases make up the majority of the atmosphere of the Earth? 21% oxygen, 78% nitrogen

- What is a chemical oxygen generator designed to do? Produce oxygen on demand

- How does OSHA define a bulk oxygen system? An assembly of equipment (storage tank, pressure regulators, safety devices, vaporizers, manifolds, and interconnecting piping) that possesses an oxygen storage capacity at normal temperature and pressure of more than 13,000 ft3

- What is an allotrope? A unique variation of an element possessing a set of physical and chemical properties that are different from the properties of any other form of the element

- Volatile organic compounds consist of which of the following? The air emissions from petroleum refineries, fuel dispensers, chemical plants, and other industrial facilities

- What are the exposure limits set by OSHA for employee exposure to ozone? To an ozone concentration in air of 0.1 ppm, averaged over an 8-hr workday

- What did the Montreal Protocol establish? It phased out the manufacture, import, and use of ozone-depleting substances.

- Hydrogen is stored by compression. as a cryogenic liquid. Both a & b

- How do hydrogen fuel cells work? Hydrogen reacts with atmospheric oxygen and creates electricity.

- A substance used to kill disease-causing microorganisms is known as a Microbicide

- What is a chlorination reaction? Chlorine atoms are incorporated into a compound in a oxidation reaction

- When chlorine is dissolved in water, what is produced? Hypochlorous acid

- What is the DOT initial isolation distance for chlorine? 3000 ft in all directions for spills from a railroad tank car, highway tank truck, or trailer.

- At what temperature does white phosphorous spontaneously ignite? 86° F

- When is red phosphorous spontaneously combustible? Only when stored in large bulk quantities

- Elemental sulfur is likely to ignite spontaneously under which of the following conditions? If exposed to an oxidizing agent

- Which of the following statement regarding responses to incidents involving white phosphorus is correct? Dams or dikes should be constructed to prevent phosphorus run- off if it begins to melt

- What agent should be used to when extinguishing a fire involving red phosphorus? Sand

- In accordance with OSHA 29 C.F.R 1910.146 an oxygen-enriched atmosphere is defined as one that contains an oxygen concentration of: More than 23.5%

- What conditions involving hydrogen increase the risk of fire and/or explosion? When released indoors or into an enclosure