# St. Petersburg College

# FFP 1111

# Chapter 8 quiz

1)What is the DOT definition of a corrosive material? Liquid or solid that causes full- thickness destruction of human skin at the site of contact within a specified period, or a liquid that chemically reacts with steel or aluminum surfaces at a rate exceeding 0.25 in./y (6.25 mm/y)

- What is an acid? Any substance that generates hydrogen ions, or H+, when dissolved in water

- Which of the following is true of mineral acids? Consist of molecules having atoms of hydrogen, an identifying nonmetal like chlorine, sulfur, or phosphorus, and sometimes oxygen

- What is an oxidizing acid? Any acid that participates in a chemical reaction as an oxidizing agent

- What is the pH scale? A numerical scale from 0 to 14 used to quantify the acidity or alkalinity of a solution with neutrality indicated as 7

- What is a basic anhydride? A metallic oxide that combines with water to produce a base

- Bases act upon the skin by changing the skin's texture into a thick, sticky material. development of deep wounds. irreversible tissue damage.All of these

- A sulfuric acid is a mineral acid that is an odorless, colorless, oily liquid having a density approximately twice that of water.

- A consumer product that destroys living tissue such as skin or eyes by chemical action defines a Corrosive substance

- OSHA limits for nitric acid include exposure to a maximum nitric acid vapor concentration of 2 ppm. 4 ppm for a short-term exposure. Both a and b

- What is hydrochloric acid used in? Certain household cleaning products, To maintain the proper acidity of the water in residential and public swimming pools, As a processing agent during the production of certain food products such as corn syrup All of these

- How is perchloric acid prepared? By distilling a mixture of potassium perchlorate and sulfuric acid at less-than-atmospheric pressure

- From what is phosphoric acid manufactured? Phosphate rock, an ore containing calcium phosphate

- What is the most commonly encountered organic acid? Acetic acid

- An aqueous solution having a pH greater than 7 is referred to as a/an Caustic

- A hydrogen ion with a pH concentration of 6 is	times greater than one having a pH equal of 7. 10

- Sodium and calcium are examples of Metals that burn in oxygen to form their corresponding metallic oxides

- Acids when reacting with metallic carbonates produce Carbon dioxide, water, and a salt of the metal

- A concentration of 70% acid and 6% to 15% nitrogen oxides with a vapor pressure of 49 mmHg at 68 F results in a material called Red fuming acid