University of Ontario Institute of Technology

ENGR 2220A

###### Quiz 5

###### Answers are highlighted

###### 1)Ceramics are:

- Hard and very tough

- Hard but not tough

- Brittle and soft

- malleable and strong

- Important properties of Metals are:

- Strong and brittle

- Strong and ductile

- Hard and non-deformable

- Stiff and non-deformable

- At temperatures below glass transition temperature, the polymers are

- Quite stiff and tough

- Quite stiff and brittle

- Exhibit large visco-elasticity

- Rubbery and ductile

- In ceramics, point defects occur in pairs so that

- mass conservation is maintained

- charge neutrality is maintained

- number of positive and negative ions are not in balance

- During three point loading test, the maximum tensile strength occurs at

- the point of maximum deflection and the outer most concave layer

- the point of maximum deflection and the outer most convex layer

- the point of maximum deflection and the central layer

- Most imperfections in solid crystal structure of metals originate:

- When it is subjected to stress and strain

- During the machining process

- When the material is heated

- When the molten metals solidify

- Vacancy diffusion is applicable to

- Substitutional impur ity atoms

- Covalently bonded molecules in atomic lattices

- Atoms present in the interstitial spaces

- Liquid molecules

- The crystalline materials deform at a much smaller force than theoretical force required to deform crystals due to the presence of:

- Twin boundaries

- Vacancies

- Impur ities

- Dislocations

- Case hardening is an example of

- Brownian diffusion

- Interstitial diffusion

- Substitutional diffusion

- Inter-molecular diffusion

- Which of the following is NOT a strengthening mechanism?

- Increasing the number of vacancies

- Formation of solid solutions

- Grain size reduction

- Inclusion of precipitates

- If a material is subjected to torsion;

- it experiences sheer stresses

- it experiences tensile stresses

- it experiences compressive stresses

- True stress on a body deformed by the application of a force is defined as:

- Force divided by the actual cross sectional area

- Force multiplied by the actual cross sectional area

- Force multiplied by the original cross sectional area

- Force divided by the original cross sectional area

- Eutectic transition occurs when

- A solid phase changes into two solid phases

- A liquid phase changes into two solid phases

- A solid phase changes into two liquid phases

- Two solid phases change into another solid phase

- When a HypoEutectic alloy is cooled from a liquid state to a solid state, the microstructure formed consists of

- Primary alpha solid granules dispersed in a eutectic microstructure

- Eutectic granules dispersed in a primary alpha solid microstructure

- Primary beta solid granules dispersed in a eutectic microstructure

- only an eutectic matrix

- The most commonly used material for fibers used for reinforcing composites is

- Boron

- Glass

- Kevlar

- SiC and Al2O3

- How are continuous fibers typically oriented in fibrous composites?

- Aligned

- Partially oriented

- Randomly oriented

- All of the above

- A big disadvantage of composites is that they are

- Brittle

- Slow and expensive to manufacture

- Unable to withstand high impacts

- Have lower strength compared to metals

- For an aligned fibrous composite, when a stress is applied perpendicular to the fibers, what is the reinforcement efficiency?

a. 0

b. 1/5

c. 3/8

d. ¾

e. 1

- For a composite material, the matrix phase is normally

- more ductile than the dispersed phase

- less ductile than the dispersed phase

- as ductile as the dispersed phase

- In composites, some times a third ingredient is used to achieve bonding of primary and secondary phases. This ingredient is called:

- Epoxy

- Coupling agent

- Super glue

- Plasticizer

- Cermets are composites in which

- Metals are held together in a ceramic matrix

- Ceramics are held together in a polymeric matrix

- ceramics are held together in a metal matrix

- polymers are held together in a ceramic matrix

- Ceramics exhibit poor fracture resistance due to

- The presence of small and omnipresent flaws or cracks

- The presence of numerous dislocations

- The inability of dislocations to move through ceramic structure

- High impurity content.

- Binary isomorphous phase diagrams depict

- The phases of multi-component systems with changing pressures and temperatures

- The phases of two component system which have unlimited solubility in each other

- The phases of two component system which have limited solubility in each other

- The phases of multi component system which have limited solubility in each other

- Binary isomorphous phase diagrams depict

- The phase changes which occur instantaneously as temperature changes

- The phase changes which occur instantaneously as pressure changes

- The phase changes which occur randomly as temperature changes

- The phase changes which occur as temperature changes very slowly