International Academy of Design & Technology

IT IT110

# Chapter 7

REVIEWING THE BASICS

1)Name two ways that a SIMM and a DIMM are alike. Name two ways they are different.

ANS: Both are rated by their speed and the amount of memory they hold, both store DRAM, and both have used EDO and FPM in the past. SIMMs hold smaller amounts of memory (from 256K to 16 MB) than DIMMs (8 MB to 1 GB). SIMMs have a single connected pin on the edge connector and DIMMs have individual pins or connectors on each side of the edge connector. SIMMs must be installed in pairs and DIMMs can be installed individually.

- How many pins are on a SDRAM DIMM? On a DDR DIMM? On a SIMM? On a RIMM?

ANS: For DIMM, 168, 184, or 240; for SIMM, 30 or 72; for RIMM, 184 or 232

- Which is likely to be more expensive, a 512-MB DIMM or a 512-MB RIMM? Why?

ANS: A RIMM is more expensive, because Rambus holds the patent on the RIMM technology and royalties must be paid by the module manufacturer.

- How many notches are on a DDR SDRAM module?

ANS: One

- What prevents a DDR DIMM from being installed in a DDR2 DIMM slot on a motherboard?

ANS: The position of the one notch on the DIMM module

- What component must be installed in every empty memory slot on a motherboard using Rambus technology?

ANS: C-RIMM

- What types of memory can be used on a 100-MHz motherboard?

ANS: SDRAM and DDR SDRAM

- Looking at an SDRAM DIMM, how can you know for certain the voltage needed by the module?

ANS: Look at the notch on the right side of the module.

- How many 30-pin SIMMs are installed in one bank?

ANS: Four

- How many 72-pin SIMMs are installed in one bank?

ANS: Two

- What are two speeds of RIMMs?

ANS: 800 MHz, 1066 MHz, 1200 MHz

- List at least four things you can do if you receive memory errors during a memory upgrade.

Possible answers are:

- Check that you have the right memory modules supported by your motherboard.

- Check that you have installed the right module size, as stated in the motherboard documentation. Verify each module that was already installed or newly installed.

- Remove and reinstall the module. Make sure it is seated in the socket at the same height as other modules.

- Remove the newly installed memory and check whether the error message disappears. Try the memory in different sockets. Try installing the new memory without the old installed. If the new memory works without the old, then the problem is that the modules are not compatible.

- Clean the module edge connectors with a soft cloth or contact cleaner. Blow or vacuum dust from the memory sockets.

- Try flashing your BIOS. Perhaps BIOS is having problems with the new memory, which can be solved by a BIOS upgrade.

- What might be a symptom in Windows of unreliable memory on a motherboard?

ANS: A general protection fault or other memory error or the system freezes or hangs.

- List at least four things you can do if you receive memory errors during normal operation when you have not recently upgraded memory.

Possible answers:

- Run a current version of antivirus software to check for viruses.

- Run diagnostic software such as PC Technician or Memtest86 test memory.

- Are the memory modules properly seated? Remove and reinstall each one. For a DIMM module, try a different memory slot.

- Look for bent pins or chips installed the wrong way on cache memory.

- Replace memory modules one at a time. For example, if the system only recognizes six out of eight megabytes of RAM, swap the last two SIMM modules. Did the amount of recognized RAM change? You might be able to solve the problem just by reseating the modules.

- Sometimes a problem can result from a bad socket or a broken trace (a fine-printed wire or circuit) on the motherboard. If so, you might have to replace the entire motherboard.

- The problem might be with the OS or applications. Download the latest patch for the software from the manufacturer’s Web site.

- If you have just installed new hardware, the hardware device might be giving an error, which is interpreted by the OS as a memory error. Try uninstalling the new hardware.

- A Windows error that occurs randomly and contains “exception fault 0E at

>>0137:BFF9z5d0” or similar text probably indicates a memory error. Test, reseat, or replace RAM.

  - If your motherboard calls for 60-ns memory, can you substitute 70-ns memory? Why or why not?

ANS: The smaller the number, the faster the memory. You should not use memory slower than the recommended memory speed.

  - When buying memory, what can you look for that might indicate that the memory is remanufactured?

ANS: If the date stamps on the chip are more than one year old, the memory is probably used.

  - Which memory module standard (RIMM or DIMM) is an open standard? Which standard is a copyrighted standard?

ANS: DIMM, RIMM

  - What is the data path size of a SIMM? A current DIMM?

ANS: 32, 64

  - What are the two current data path sizes of RIMMs?

ANS: 16, 32

  - What improvements did DDR make over regular SDRAM?

ANS: Data is processed twice in one clock beat, doubling the speed of regular SDRAM

  - When a DIMM has chips on both sides of the module, do the pins on one side of the module work independently or dependent to pins on the other side of the module?

ANS: Pins on each side work independently.

  - Which is faster, CL3 memory or CL5 memory?

ANS: CL3 is faster than CL5 memory.

  - You are looking to purchase two DIMMs running at 400 MHz. You find DIMMs advertised at PC4000 and PC3200. Which do you purchase?

ANS: PC3200

  - You need to find out how much RAM is installed in a system. What command do you enter in the Run dialog box to launch the System Information utility?

ANS: Msinfo32

  - Although ECC memory costs more than non-ECC memory, why would you choose to use it?

ANS: ECC memory is more reliable than non-ECC memory.

# THINKING CRITICALLY

- If your motherboard supports DIMM memory, will RIMM memory still work on the board?

ANS: No, you can only use the type of memory module the board is designed to support.

- If your motherboard supports ECC SDRAM memory, can you substitute SDRAM memory that does not support ECC? If your motherboard supports buffered SDRAM memory, can you substitute unbuffered SDRAM modules?

ANS: You can substitute non-ECC memory on an ECC board, and the error-checking feature will be shown disabled in CMOS. You cannot use unbuffered SDRAM on a motherboard that supports buffered memory, because the notches on buffered DIMMs are in different positions than for unbuffered DIMMs.

- You have just upgraded memory on a computer from 64 MB to 128 MB by adding one DIMM. When you first turn on the PC, the memory count shows only 64 MB. Which of the following is most likely the source of the problem? What can you do to fix it?

- Windows is giving an error because it likely became corrupted while the PC was disassembled.

- The new DIMM you installed is faulty.

- The new DIMM is not properly seated.

- The DIMM is installed in the wrong slot.

ANS: The new DIMM is not properly seated. Turn off the PC and reseat the DIMM. Check that the DIMM is standing in the slot at the same height of the other DIMMs installed.

- Your motherboard supports dual channeling and you currently have two slots used in Channel A on the board. You want to install an additional 512MB of RAM. Will your system run faster if you install two 256 MB DIMMs or one 512 MB DIMM? Explain your answer.

ANS: The system will run faster if you install two DIMMs because dual channeling can be used if both Channel B slots are filled. Dual channeling is faster than single channeling.