International Academy of Design & Technology

IT IT110

# Chapter 9 Reviewing the Basics

# ANSWERS ARE RED

1)Identify three things that may cause monitor flicker.

Poor cable connections, accumulated magnetic fields, electrical noise, vertical scan frequency below 60 Hz, screen resolution set too high

- Describe what to do if you have just spilled soda pop on your keyboard.

Replace the keyboard. You might be able to salvage the keyboard by thoroughly rinsing it in running water.

- Explain how to check that chips on a video card are properly seated in their sockets.

Remove the card from the expansion slot and then use a screwdriver to press down firmly on each corner of each socketed chip on the card.

- When troubleshooting problems with a monitor in Windows XP, why would you enter Safe Mode?

To allow the OS to select a generic display driver and low resolution

- Describe how to boot Windows XP into Safe Mode.

Press F8 during startup to display the startup menu. Then use the arrow keys to select Safe Mode from the startup menu.

- Why would an external modem cost more than an internal modem?

Because an external modem has the added cost of an external case, power source, and data cable

- Name three possible ways a scanner might interface with a motherboard.

Through a SCSI port, a USB port, or through FireWire

- By definition, what system resources does COM1 use? COM2? COM3? COM4?

COM1—IRQ 4, memory addresses 03F8-3FF COM2—IRQ 3, memory addresses 02F8-2FF COM3—IRQ 4, memory addresses 03E8-3EF COM4—IRQ 3, memory addresses 02E8-2EF

- What is the display resolution for standard VGA settings?

640 x 480

- What is the maximum length of a serial cable?

50 feet

- To what does RS-232 refer?

It refers to Reference Standard 232 revision c. This is the standard for the serial port, which is sometimes called the RS-232 port.

- How many pins are on a typical serial port?

9

- What is the name of a barcode reader that is a gun-type reader that can be handheld and is often used at a checkout counter?

CCD scanner

- What is the purpose of a keyboard wedge?

A keyboard wedge is used so that a keyboard and barcode reader can share a keyboard port.

- What is the name of the technology within the chipset that controls the speed of serial ports?

UART

- Why might you choose to use ECP mode for your parallel port rather than EPP mode?

If you need greater speed

- When might you need to disable ECP mode for a parallel port?

If you are having problems with resource conflicts

- How would you disable a serial port on a motherboard?

Use CMOS setup

- What Windows OSs support USB? Include the OS version numbers where that information is important.

Windows 95 OSR 2.1 was the first Microsoft OS to support USB, although Windows 98 offers much improved USB support. In addition to Windows 95 with the USB update and Windows 98, Windows 2000 and Windows XP support USB, but Windows NT does not.

- What is the maximum speed of Original USB? Of Hi-Speed USB?

12 Mbps, 480 Mbps

- What is the maximum length of a USB Hi-Speed cable?

Five meters

- What are two other names for FireWire? What is the highest data throughput approved for FireWire?

- Link and IEEE 1394; data throughput is 3.2 Gbps

- When selecting a motherboard, why would you prefer to use PCI Express for the video card rather than AGP?

Because PCI Express x1 is faster than AGP

- What is the most important disadvantage of using biometric devices to authenticate a person to have access to a system?

The danger of false negatives or false positives

- What criteria affect how much video RAM is needed for a video card to hold one frame buffer?

Screen resolution, color depth, and enhancements to color information (alpha blending)

- Give three examples of monitor screen sizes. How are monitor screen sizes measured?

There are 14-, 15-, 17-, and 21-inch screen sizes. Screen size describes the diagonal length of the screen surface.

- What are two advantages of setting an LCD monitor to run in its native resolution?

Faster response time and sharper display

- What type of CRT monitor can offer a variety of refresh rates?

Multiscan

- How many pins are used in the IEEE 1394 connector that supports the IEEE 1394b standard?

Nine

- How many keyed notches does an AGP universal slot have?

None

- What makes a device an ergonomic device?

If it is designed for safe and comfortable interaction between human beings and machines

- How many pins are there on a DIN connector and a PS/2 connector for a keyboard?

Five pins on a DIN connector; six pins on the smaller PS/2 connector

- What three colors are used to build all colors on a color monitor screen?

Red, green, and blue

- Which gives better image quality, a .25-mm dot pitch monitor or a .28-mm dot pitch monitor? Why?

A .25 does because the smaller the pitch, the sharper the image.

- If a mouse begins to be difficult to operate, what simple thing can you do to help?

Remove the cover to the mouse ball and the mouse ball itself, and clean the rollers with a cotton swab dipped in a very small amount of liquid soap.

# Thinking Critically

  - You plug a new scanner into a USB port on your Windows XP system. When you first turn on the scanner, what should you expect to see?

- You see a message displayed by the scanner software telling you to reboot your system.

- You see the Found New Hardware Wizard launch.

- Your system automatically reboots.

- You see an error message from the USB controller.

The Found New Hardware Wizard launches.

  - You install the software bundled with your digital camera to download pictures from your camera to your system using a serial port. Next you plug up the camera to the port using a serial cable and turn on your camera. You attempt to use the software to download pictures, but the software does not recognize the camera is present. What do you do next?

- Return the camera and purchase one that uses a USB port for downloading.

- Reinstall the bundled software.

- Access CMOS setup and verify that the serial port is enabled.

- Use Device Manager to verify that the OS recognizes the serial port.

- Replace the serial cable.

Follow the troubleshooting rule: Do the simple things first. Therefore, the first thing to do is to use Device Manager to verify that the OS recognizes the serial port.

  - You turn on your Windows 2000 computer and see the system display POST messages.

Then the screen turns blue with no text. Which of the following items could be the source of the problem?

    - The video card

    - The monitor

    - Windows

    - WordPerfect software installed on the system Windows could be the problem, but not the other three items.