International Academy of Design & Technology

IT IT110

# Chapter 10

REVIEWING THE BASICS

ANSWERS ARE RED

1)What must be true before MMX, SSE, SSE2, and 3DNow! technology can improve multimedia performance on a PC?

The applications and hardware devices must be designed to use the technology.

- What is the significance of the multi-session feature on a CD drive?

The drive can read a disc that has been created in multiple sessions rather than having been written all at once.

- Name three ways a CD drive can interface with a motherboard.

With an EIDE interface, sharing an IDE connection, a cable, or both with a hard drive. These drives use the ATAPI standard.

Using a SCSI interface with a SCSI host adapter

For portable drives, with an external port (such as a USB, FireWire, or SCSI port) on your PC

- Which side of a CD contains data?

The bright, bottom side (not the side with the label)

- If a CD drive and a hard drive are sharing the same data cable in a computer system, what type of connection is the CD drive using? Which of the two drives should be set to master? Which to slave?

The CD drive is using an EIDE connection. The hard drive should be set to master and the CD drive to slave.

- What unit of measure is used to express the sampling rate of a sound card?

Hertz (Hz)

- Why must sound and video input into a PC be converted from analog to digital?

Analog data is smooth and continuous, whereas digital data, such as the binary data used in computers, has distinct gradations. The process of producing digital data by measuring data at a series of representative points is called sampling.

- What is the sampling rate (in Hz) of music CDs?

44,100 Hz

- How many samples can be stored in 8 bits?

256

- What would be a quick, short test to see if a sound card was successfully installed?

Play a music CD.

- In a system that uses a CD drive, the audio wire connects the	to the	.

CD drive, sound card

- Why would you want to re-tension a backup tape?

Re-tensioning fast-forwards and rewinds the tape to eliminate any loose spots. Some backup software requires this.

- Which holds more data, a Microdrive or a Zip drive?

A Microdrive

- How is the direction of data flow different for data transfers for MP3 players and digital cameras?

MP3 files are downloaded from the PC to the MP3 player, in contrast to a digital camera, for which data is transferred or uploaded from the camera to the PC.

- What is the significance of Sound Blaster compatibility for a sound card?

A Sound Blaster-compatible card can understand the commands sent to it that have been written for Sound Blaster.

- When using a single speaker, which port on a sound card is used to send sound out?

The front speaker port, which is generally the center port and is often green

- What is the difference between MPEG, JPEG, and MP3? Explain for what each one is used.

MPEG (Moving Pictures Experts Group) is a standard for data compression for motion pictures, video, and audio. JPEG is a standard for data compression for photographs and graphics. MP3 is a popular version of MPEG that is used for data compression for audio.

- Name at least four features you should look for when buying a video capture card.

- A FireWire interface

- Data transfer rates

- Capture resolution and color-depth capabilities

- Ability to transfer data back to the digital camcorder or VCR

- Stereo audio jacks

- Video-editing software bundled with the card

- When connecting cords to the rear of a sound card and the card has a blue, red, yellow, and black port, which port is most likely to be used for the microphone?

Red port

- In the question above, which port is likely to be used for sound input from a DVD player sitting beside the computer?

Blue port

- What are the three ways that data on a DVD can be decoded?

- Decoder software is installed on the hard drive.

- A decoder card is installed in an expansion slot.

- A decoder is part of the firmware on a video capture card.

- What is the most popular way an internal DVD drive interfaces with a motherboard?

EIDE connection

- What is the difference between CD, CD-R, and CD-RW drives?

A CD-ROM is read-only. A CD-R is recordable once. A CD-RW is rewriteable, meaning that you can write new data over old data.

- Rank these storage methods in order of their storage capacity: DVD, floppy disk, CD, tape.

From least to greatest: floppy disk, CD-ROM, DVD, tape

- How many unique speakers can Surround Sound 7.1 support?

Up to 8 speakers

- Which type of flash memory device is typically used on a Sony digital camera?

Memory Stick

- What is currently the most popular memory device used in digital camera?

Secure Digital (SD) cards

- Of the flash memory device used in digital cameras, which uses the latest technology and holds the most data?

xD-Picture Card

- Current DVD recordable drives for personal computers can write only 8.5 GB of data on a DVD. How many layers and how many sides or surfaces of the disc are used for the data?

Data is written in two layers to one side or surface of the disc.

- Which version of RAID is supported by Windows XP? Does this RAID version provide fault tolerance?

RAID 0 (striped), No

# THINKING CRITICALLY

- You have just installed a new sound card and its drivers and connected the speakers and amplifier. You insert a music CD into the drive to test the drive. Windows Media Player launches and says it is playing the CD, but you don’t hear music. What do you do first?

  - Check the volume controls on the speaker amplifier.

  - Check the connections of the amplifier and speakers to the card.

  - Check Device Manager for errors with the sound card.

  - Verify that the amplifier has power.

Do the simple thing first: Check the volume controls on the speaker amplifier.

- You have just upgraded your computer from Windows 98 to Windows XP. Now your system has no sound. What are the first two things you do?

  - Check Device Manager to see if the sound card is recognized and has no errors.

  - Reinstall Windows 98.

  - Use Device Manager to uninstall the sound card.

  - Identify your sound card by opening the case and looking on the card for manufacturer and model.

  - Identify your sound card by finding the documentation and driver CD that came with the card.

  - Download Windows XP drivers for the sound card from the sound card manufacturer’s Web site.

Check Device Manager to see if the sound card is recognized and has no errors. Then identify your sound card by finding the documentation and driver CD that came with the card.

- You have just installed a new DVD drive and its drivers, but the drive does not work.

You check the power and data cables and feel comfortable that the hardware installation is correct. You then decide to reload the device drivers. What is the first thing you do?

- Open the Control Panel and launch the Add New Hardware Wizard.

- Open Device Manager and choose Update Driver.

- Remove the data cable from the DVD drive so Windows will no longer recognize the drive and allow you to reinstall the drivers.

- Open Device Manager and uninstall the drive.

Open Device Manager and uninstall the drive.

- Which method of fault tolerance is the least expensive per MB of storage, disk duplexing or disk striping with parity? Explain your answer.

Disk duplexing requires writing the same data twice and requires an extra controller. Disk striping with parity only writes the data once and requires only one controller. Although the parity information in disk striping with parity does take up some space, it does not take up as much space as the duplicate data in disk duplexing. Therefore, disk duplexing is more expensive.

- Does RAID 0 provide fault tolerance? Explain your answer.

RAID 0 does not provide fault tolerance because a single logical drive is spread over two or more physical hard drives. If one fails, the data cannot be recovered from the others. RAID 0 is designed to increase performance, not provide fault tolerance.