International Academy of Design & Technology

IT IT110

ANSWERS ARE RED

Chapter 17

REVIEWING THE BASICS

1)Name three types of Ethernet. What transmission speed does each support?

10-Mbps Ethernet (10 Mbps), Fast Ethernet (100 Mbps), and Gigabit Ethernet (1000 Mbps)

- What is the maximum length of a cable on a 100BaseT network?

- meters (328 feet)

- What does the 100 in the name 100BaseT indicate?

The transmission speed (100 Mbps)

- What is broadcasting? Name a network connection device that uses it and one that does not.

Broadcasting occurs when a device sends packets to every device to which it is connected, regardless of the packet’s destination. Hubs use broadcasting, and switches do not.

- What IEEE standards describe Ethernet? What IEEE standards describe wireless LAN?

Ethernet: 802.3

Wireless LAN: 802.11, 802.11b, 802.11a, 802.11g, 802.11d

- What are three ways a wireless network can be secured?

Filter MAC addresses, use WEP encryption, and use a VPN.

- What connecting device do you use for a small LAN? Two or more connected networks?

For a small network, a switch is best. To connect two or more networks, a router is required.

- Describe the structure of an IP address. How is it different from a MAC address?

An IP address is a series of four numbers, each no larger than 255, separated by periods. A MAC address is expressed as six pairs of hex numbers separated by dashes. A MAC address cannot change, but an IP address can change on a computer; a MAC address is like a local address; and an IP address is like a long-distance address.

- How many potential IP addresses are there?

4.3 billion

- How many networks and addresses are available for Class A IP addresses? Class B?

Class C?

Class A: 127 networks; 16 million addresses Class B: 16,000 networks; 65,000 addresses Class C: 2 million networks; 254 addresses

- Why are Class D and E addresses not available to individuals and companies?

Class D addresses are used for multicasting, and Class E addresses are reserved for research.

- Which octets are used for the network address and for host addresses in Class A? Class B? Class C?

Class A: N.H.H.H Class B: N.N.H.H Class C: N.N.N.H

- In what class is the IP address 185.75.255.10?

Class B

- In what class is the IP address 193.200.30.5?

Class C

- Describe the difference between public and private IP addresses. If a network is using private IP addresses, how can the computers on that network access the Internet?

Public IP addresses are available for use on the Internet. Private IP addresses are used on private intranets that are isolated from the Internet. Networks that use private IP addresses can access the Internet through a proxy server, which substitutes its IP address for the address of the device trying to access the Internet.

- Why is it unlikely that you will find the IP address 192.168.250.10 on the Internet?

Because this address is in one of the IP address ranges that RFC 1918 recommends be reserved for private networks.

- Which Windows operating system does not automatically include the NetBEUI protocol?

Windows XP

- What are the two ways an IP address can be assigned to a PC? What is one advantage of each?

IP addresses can be assigned through static addressing (in which IP addresses are permanently assigned to workstations) or dynamic addressing (in which IP addresses are assigned for the current session only). An advantage of static IP addressing is that it does not require a DHCP server. An advantage of dynamic IP addressing is that fewer IP addresses than the total number of workstations can satisfy the needs of a network.

- For what are the Ping, Ipconfig, and Winipcfg utilities used?

The Ipconfig (on Windows 2000/XP) and Winipcfg (on Windows 9x) commands are used to display a machine’s IP address and the MAC address of its NIC. The Ping command is used to send a signal to a remote computer and test connectivity.

- Of IPX/SPX, TCP/IP, and NetBEUI, which is routable and which is not?

IPX/SPX and TCP/IP are routable, and NetBEUI is not.

- Place the following bandwidth technologies in the order of their highest speed, from slowest to fastest: SDSL, ISDN, regular telephone lines, cable modem, Gigabit Ethernet, ATM.

Regular telephone lines, ISDN, SDSL, cable modem, ATM, Gigabit Ethernet

- When using DSL to connect to the Internet, the data transmission shares the cabling with what other technology?

Telephone

- Give two examples of broadband technology.

Possible answers: cable modem and DSL

- Which is more expensive, UTP CAT5e cabling or STP CAT5e cabling?

STP CAT5e

- What is the most common type connector used with coaxial cable on a ThinNet Ethernet network?

BNC

- If you wanted to upgrade your 100BaseT Ethernet network so that it will run about 10 times the current speed, what technology would you use?

Gigabit Ethernet

- A network uses a subnet mask of 255.255.255.0 and has a computer on the network with the IP address of 120.12.12.3. Will the IP address 120.12.10.3 be on the network? Explain your answer.

No, the first three octets must match and they do not.

- What networking protocol was first used by Novell NetWare and is not supported on the Internet?

IPX/SPX

- If you were going to connect two PCs together in a simple network using the network ports on each PC, what type cable would you use?

A crossover cable

- When using a cable modem to connect to the Internet, the data transmission shares the cabling with what other technology?

Television

# THINKING CRITICALLY

  - You have just installed a network adapter and have booted up the system, installing the drivers. You open My Network Places on a remote computer and don’t see the computer on which you just installed the NIC. What is the first thing you check?

    - Is File and Printer Sharing installed?

    - Is the NetBEUI protocol installed?

    - Are the lights on the adapter functioning correctly?

    - Has the computer been assigned a computer name?

- Are the lights on the adapter functioning correctly?

  - You work in the Accounting Department and have been using a network drive to post Excel spreadsheets to your workgroup as you complete them. When you attempt to save a spreadsheet to the drive, you see the error message, “You do not have access to the folder ‘J:\’. See your administrator for access to this folder.” What should you do first? Second?

    - Ask your network administrator to give you permission to access the folder.

    - Check My Network Places to verify that you can connect to the network.

    - Save the spreadsheet to your hard drive.

    - Using Windows Explorer, remap the network drive.

    - Reboot your PC.

c. Save the spreadsheet to your hard drive.

b. Check My Network Places to verify that you can connect to the network.

  - Your job is to support the desktop computers in a small company of 32 employees. A consulting firm is setting up a private Web server to be used internally by company employees. The static IP address of the server is 192.168.45.200. Employees will open their Web browser and enter personnel.mycompany.com in the URL address box to browse this Web site. What steps do you take so that each computer in the company can browse the site using this URL?

Make this entry in the Hosts file on each computer:

192.168.45.200 personnel.mycompany.com

  - Linda has been assigned the job of connecting five computers to a network. The room holding the five computers has three network ports that connect to a switch in an electrical closet down the hallway. Linda decides to install a second switch in the room. The new switch has four network ports. She uses a crossover cable to connect one of the four ports on the switch to a wall jack. Now she has five ports available (two wall jacks and three switch ports). While installing and configuring the NICs in the five computers, she discovers that the PCs connected to the two wall jacks work fine, but the three connected to the switch refuse to communicate with the network. What could be wrong and what should she try next?

Try connecting one of the non-working computers to a wall jack. If that works, then the problem is the switch or the cable connecting the switch to the wall jack. Try using a patch cable for the connection.