# ITT Tech

# NT 1210

# Chapter 10

1)Which of the following answers lists a need of transport layer protocols that can be met by the TCP/IP network layer protocols? a. A delivery service that recovers and resends lost data b. Segmenting data into smaller chunks of a size that can be sent through the TCP/IP network c. Delivering data to the correct destination application process on the destination host

d. Delivering data to the correct destination host e. Providing communications functions to application software

Answer: D. The transport layer needs an IP packet delivery service. That service delivers IP packets from one host to the next. The rest of the answers are either services provided by the transport layer protocol to the application layer protocol, or services provided by the application layer protocol to the application.

- Which of the following answers list a need of application layer protocols that can be met by the TCP/IP transport layer protocols? (Choose three answers.) a. A delivery service that recovers and resends lost data b. Segmenting data into smaller chunks of a size that can be sent through the TCP/IP network c. Delivering data to the correct destination application process on the destination host d. Delivering data to the correct destination host e. Providing communications functions to application software

Answer: A, B, C. The application layer protocols have a variety of needs because many application layer protocols exist. All the correct answers list features provided to the application layer by the TCP protocol in particular. The other answers list either services provided by the network layer protocols to the transport layer protocols, or services provided by the application layer protocol to the application.

- Which of the following answers lists a service provided by an application layer protocol to the application? a. The ability to identify the correct destination application process on the other end of an application flow b. The ability to communicate using a TCP/IP network c. Encoding data over a cable d. Encapsulating data between a header and trailer

Answer: B. Application layer protocols enable an application to send and receive data in a TCP/IP network. The application layer protocols rely on the functions of other layers to identify the right destination application process (transport layer) and encode the data over cables (physical layer).

The application layer protocol can encapsulate data behind a header, but only the data-link protocols encapsulate data between a header and trailer.

- A user opens email client software on her tablet computer after connecting to the Internet. The user successfully downloads her latest email. Another student happens by and starts spouting off about how the first student has been using email, POP3, IP, TCP, HTTP, 802.11, and other acronyms all just to check email. The first student just stares at this second student, but it does cause the first student to wonder: Which of the following answers accurately describes where some of these features are implemented on the tablet? a. POP3 in the email application software b. HTTP in the email application software c. TCP in the email application software d. UDP in the OS

Answer: A. Email clients can check their email using POP3, which in turn uses TCP, which in turn uses IP. The email client application typically implements the application itself, plus the application layer protocol (POP3 in this case). The operating system (OS) typically provides the TCP and IP functions.

- A user of a tablet computer goes to a coffee shop and connects to the Internet through a wireless LAN. The user then connects to a web server somewhere far away in the Internet. Which of the following statements are true about the messages that flow from the tablet’s web browser toward the web server? (Choose two answers.) a. The 802.11 frame sent by the tablet arrives at the web server. b. The IP packet sent by the tablet arrives at the web server. c. The TCP segment sent by the tablet arrives at the web server. d. The UDP segment sent by the tablet arrives at the web server.

Answer: B, C. TCP/IP networks use data-link headers and trailers to deliver data across a single data link to the next router in a route. Because the tablet sits far away from the web server, many routers exist between the tablet and the web server. As a result, the data-link header and trailer originally created by the tablet will be discarded. However, the IP packet created by the tablet, and the encapsulated TCP segment, will arrive at the destination web server. HTTP, the application protocol used by web browsers and web servers, does not use UDP, so the tablet will not create a UDP datagram.

- Which of the following answers accurately describe the size of the TCP, UDP, and IP headers, assuming that the headers include no additional optional fields? (Choose three answers.) a. IP: 20 b. TCP: 8 c. UDP: 8 d. TCP: 20 e. UDP: 12

Answer: A, C, D. Both IP and TCP support optional additional fields, but without these options, the header lengths are IP (20), TCP (20), and UDP (8).

- When thinking about the messages that flow inside a TCP/IP network, you can think about the header for a particular layer and its encapsulated data, ignoring any lower-layer headers and trailers. Which of the following answers list a protocol with the correct matching term for the name of the message that includes that protocol’s header plus its encapsulated data? (Choose two answers.) a. UDP: Packet b. TCP: Connection c. UDP: Datagram d. TCP: Segment e. UDP: Flow

Answer: C, D. Per this chapter, TCP messages are called segments and UDP messages are called datagrams. Earlier chapters defined IP’s messages as packets.

- An application running on a user’s device is currently sending data to/from an application on some server inside an enterprise TCP/IP network. According to this chapter, which of the following terms are the most common terms that refer to the idea of these two application processes communicating with each other? (Choose two answers.) a. Connection b. Pipe c. Circuit d. Flow

Answer: A, D. The term flow can be used regardless of transport layer protocol, while the term connection is typically used only for TCP. The term session can also be used at times, as a holdover from the old OSI terminology with the OSI session layer.

- Which of the following answers describes a commitment that both TCP and UDP make to the application protocols that use them? a. Delivering data to the correct destination host b. Delivering data in the same order in which it was sent c. Delivering data to the correct destination application process d. Segmenting large data into sizes that flow over the network

Answer: C. Both TCP and UDP commit to the application protocol to deliver the data to the correct application process on the destination host. IP commits to deliver the data to the correct host, as a service to TCP and UDP. Only TCP, and not UDP, offers in-order delivery and data segmentation.

- Which of the following answers lists a transport protocol with the correct size, in bits, of that protocol’s destination port field in its header? a. UDP: 8

b. TCP: 12 c. UDP: 16 d. TCP: 32

Answer: C. Both the TCP and UDP headers begin with a source and destination port field. Each field is consistently 2 bytes (16 bits) long.

- A user of a tablet computer has three browser tabs open, with those tabs currently using TCP port numbers 55555, 55556, and 55557. The user opens another browser tab. Which of the following port numbers could the browser use? a. 55555 b. 55558 c. 80 d. 77777

Answer: B. The new tab, in a web browser, will use TCP, because web browsers use HTTP, which uses TCP. Because TCP port 55555 is already in use on the tablet, the tablet cannot use 55555 for the new tab in the browser. 55558, unused before the user open this new tab, could be used. The tablet would not use a number from the well-known port number range (0–1023). Also, port numbers can be as high as 65535, but no higher, so 77777 would not work either.

- A user of a tablet computer has three browser tabs open, with those tabs currently using TCP port numbers 55555, 55556, and 55557. The user opens an application that uses UDP. Which of the following port numbers could the new application use? (Choose two answers.) a. 55555 b. 55558 c. 80 d. 77777

Answer: A, B. UDP and TCP ports are separate sets of numbers and only need to be unique on a host within that set. In other words, the fact that the tablet already uses three TCP ports in no way prevents the tablet from using those same numbers as UDP ports. As a result, both 55555 and 55558 can be used. The tablet would not use a number from the well- known port number range (0–1023). Also, port numbers can be as high as 65535, but no higher, so 77777 would not work either.

- The user of host A opens a web browser and successfully loads a web page hosted on web server B. Host A’s browser uses TCP port 55555, and web server B uses the well-known HTTP port for web servers. In the messages that contain the contents of the web page that appeared in the web browser’s window, which of the following answers is true? a. The source port = 80. b. The source port = 55555. c. The destination port = 80.

d. The question gives too little information to determine whether the other answers are correct.

Answer: A. The content of the web page must flow from the server to the client (browser). The server, using HTTP well-known port 80, will send messages with its own port number (80) as the source port number. The destination port number will identify the application port number on the client, in this case 55555.

- The user of host A opens a web browser and successfully loads a web page hosted on web server B. Host A’s browser uses TCP port 55555, and web server B uses the well-known HTTP port for web servers. Which of the following terms refers to the category or type of port number used by the web browser (55555 in this case)? a. Plain old port b. Registered port c. Simple port d. Dynamic port

Answer: D. Clients dynamically allocate a dynamic port, always avoiding well-known port numbers and sometimes avoiding the registered port number ranges as well. Two of the incorrect answers, plain old port and simple port, are not used as real terms related to port numbers.

- Which of the following answers lists the correct transport protocol and well-known port number used by each application protocol listed? a. DNS– UDP: 53 b. HTTP–UDP: 80 c. Telnet–TCP: 110 d. POP3–UDP: 53

Answer: A. DNS uses both TCP and UDP, port 53 in both cases, but uses UDP for name resolution messages. HTTP uses TCP port 80. Telnet uses TCP port 23. POP3 uses TCP port 110.

- Which of the following statements are true regarding MTU? (Choose two answers.) a. Defines the maximum size of an IP packet that can flow from host to host over the entire IP route. b. The MTU value describes the size of the entire IP packet. c. Defines the maximum size of an IP packet

that can be sent out a particular interface. d. The value describes the size of the entire IP packet’s data field.

Answer: B, C. The IP MTU defines a value, per interface, that is the maximum length of the entire IP packet that the node can send out that interface. The value describes the entire IP packet, which includes the IP header. It does not define the size of the IP packet that can flow over the entire route.

- Which of the following statements are true regarding MSS? (Choose two answers.) a. Defines the maximum size of a TCP segment that can flow from host to host over the entire IP route. b. The MSS value describes the size of the entire TCP segment. c. Defines the maximum size of a TCP segment that can be sent out the interface on the source host. d. The value describes the size of the entire TCP segment’s data field.

Answer: C, D. The TCP MSS defines a value, per interface, that is the maximum length of the entire TCP segment that the TCP software can build on that node and send out that interface. The value describes the data field only of the TCP segment. The value defines the size of the segment created on the sending node and does not restrict the nodes in the TCP/IP network, including the routers that ignore TCP when forwarding IP packets.

- A web server hosts a web page. Part of that web page includes a graphics image, in a file named fred.jpg, that shows a photo of Fred. The file size is 300 KB. The web server’s Ethernet interface has an IP MTU of 1500. Which of the following answers is true about the process that moves the contents of fred.jpg from the web server to the web browser? a. HTTP on the web server breaks the data mostly into 1500-byte chunks. b. TCP on the web server breaks the data mostly into 1480-byte chunks. c. IP on the web server breaks the data into mostly 1480-byte chunks. d. TCP on the web server breaks the data into mostly 1460-byte chunks.

Answer: D. With an IP MTU of 1500, the MSS will be 1460 (MTU – 40, with 40 being the size of the IP and TCP headers). HTTP does not segment the data, relying on TCP to do the work.

- Which of the following answers best describes how TCP guarantees delivery of data over a TCP connection? a. By sending two copies of every TCP segment b. By sending a TCP checksum that allows the receiving host to correct any errors that occur during transmission c. By noticing when data is lost and then resending the lost TCP segments d. By routing packets over links that have the fewest number of errors

Answer: C. TCP uses an error recovery process, using the TCP Sequence (SEQ) and Acknowledgment (ACK) fields, to let the hosts signal when data arrives correctly, and when it does not.

- Which of the following answers list a feature that TCP offers but that UDP does not? (Choose two answers.) a. In-order delivery of data b. Identifying servers using well-known ports c. Delivering data to the correct application processes d. Error recovery

Answer: A, D. TCP offers all the services listed in the answers. However, two of the answers list features also offered by UDP.