# ITT Tech

# NT 1210

Chapter 7

1)A user of a home telephone picks up her phone and makes a telephone call to a friend’s home telephone in another part of town. Which of the following is likely to be true about this call? a. It uses a single pair of wires on the local loop at each end of the call. b. It uses a telco service called packet switching. c. It uses digital circuits at least on the part of the call from the CO to the phone that made the phone call. d. None of the answers is correct.

Answer: A. The local loop cabling from the CO to most home telephones uses a single twisted pair of wires, an analog circuit, with the telco performing circuit switching to switch the analog circuit through the telco.

- Which of the following are services that telcos have offered as WAN services over the years? (Choose two answers.) a. Switched analog circuits b. Dedicated digital circuits c. Partition switching d. Telekinetic- division multiplexing

Answer: A, B. The two correct answers list legitimate WAN services that are also mentioned in this chapter. The two incorrect answers were simply made up as similar variations of packet switching and time-division multiplexing.

- This chapter claims that IP routers work well as devices that connect to many different types of interfaces, including LANs and WANs. Which of the following answers list the reasons why routers do well in this role? (Choose two answers.) a. Routers have much faster CPUs and can do the hard

work to translate the incoming data-link header into the outgoing data-link header format. b. Router vendors sell many different types of physical interface cards, so the routers have the right physical connectors to connect to different types of networks. c. Routers use Layer 3 and ignore Layers 1 and 2. d. Routers discard old data-link headers and insert new data-link headers as part of their logic, which treats each interface’s lower- layer details as independent from each other.

Answer: B, D. Routers have to support Layers 1, 2, and 3 to do their work, focusing on Layer 3, but using Layers 1 and 2, both LAN and WAN, to deliver IP packets. A side effect of that process is that routers discard the data-link header/trailer of incoming frames and add a new header/trailer to outgoing frames. Also, router vendors sell many types of router interface cards because the router needs to connect to many type of interfaces.

- An enterprise network has many routers that connect to both a LAN as well as the WAN. Which one of the following statements best describes how routers typically use and think about their WAN connections? a. As a destination where end-user hosts reside b. As a transport service to deliver IP packets to the next IP router c. As nothing, because routers ignore Layers 1 and 2 d. As an interesting and complex technology-filled network of its own, with such features as circuit switching, packet switching, SONET, ATM, and others

Answer: B. The question specifically asks for the one best answer and for the routers’ perspective. In this case, the answer about routers ignoring Layers 1 and 2 is simply incorrect; routers must know about these layers to receive and send data. Routers can send IP packets to user host destinations on the WAN itself, but because most user devices sit on a LAN of some kind, the most typical case for routers is to view the WAN as a way to move IP packets to another router. Finally, the answer about the interesting technology in the WAN might be true about a human’s perspective, but the router typically has no or little idea of the technology used inside the WAN.

- An enterprise builds a WAN design on paper. The main site, where all the servers sit, is shown in the center of a network diagram. The WAN has a leased line from that site to every remote site, with the remote sites

drawn around the edges of the drawing in a big circle. None of the remote sites have a leased line directly between them. Which of the following terms is most typically used to describe this WAN topology? a. Multipoint b. Star c. Point-to-point d. Hub-and-spoke

Answer: D. Although in a LAN the same drawing would be called a star topology, in WANs, this topology typically goes by the name hub-and- spoke, like the hub and spokes of a bicycle wheel.

- A U.S.-based company wants to order a leased line between two sites, with the leased line using the traditional T-carrier DS0, DS1, and DS3 types of lines. Which of the following answers describe the available speeds and distances for the leased line? (Choose two answers.) a. No longer than 62 miles (100 kilometers) for up to T1 speed b. No faster than 43.736 Mbps (T3) c. 64 Kbps, 1.544 Mbps, and any speed in between d. No literal distance limitation

Answer: B, D. The leased lines have no literal distance limitation, because they physically exist as multiple separate segments. The only limit to the distance relates to where the telco has the physical links available, and how much the customer is willing to spend. Because the question limited the discussion to DS0, DS1, and DS3, the speed limitations do reach up to T3 speed (43.736-Mbps line rate). However, the speeds cannot be just any speed between DS0 (64-Kbps) and T1 (1.544-Mbps) speeds.

- A telco customer has purchased a T1 leased line between two sites. The customer has already bought a router for one site, with a serial interface card with a built-in CSU/DSU. Which of the following answers lists other physical parts that the customer will need for that site’s installation of the T1 leased line? a. A patch cable with RJ-48 connectors b. A serial cable c. An external CSU/DSU d. A demarc

Answer: A. Because the router serial interface card already has a CSU/ DSU in it, the site does not need an external CSU/DSU, and it does not need a serial cable to connect the router to an external CSU/DSU. The router interface does need to be cabled to the socket supplied by the telco on the end of the T1 line, most often with a patch cable that uses RJ-48 connectors. Finally, the demarc is not a device or plug, but a conceptual

dividing line between the customer’s responsibilities and the telco’s responsibilities.

- A telco customer has a T1 leased line between two sites, called sites A and B. The telco has several switches between sites A and B that use T3 trunks. These switches use the T3 trunks along with time-division multiplexing (TDM) to create the leased line from site A to B. Which of the following answers is true about how TDM works in this design? a. The switches map the T1 customer lines to a T1 channel inside the T3 trunks. b. The switches interpret the customer bits as packets and forward the packets to the correct destination over the T3 trunks. c. The switches map the T1 customer lines to one of 28 different frequencies in the T3 trunks. d. The switches perform a digital-to-analog (D/A) conversion and send the bits over a unique analog frequency over the T3 trunks.

Answer: A. TDM in the telco T-carrier system takes the bits from slower- speed lines, like T1s, and maps them to a time channel inside a higher- speed line, like a T3. One answer lists the logic of a packet switch, where the packet switch makes a forwarding decision based on an address, but TDM switches are circuit switches, switching the circuit from end to end. The T-carrier system and TDM technology do not divide the links into different frequencies, but different time periods, called channels.

- A telco network has TDM switches in the CO, ready to support T1 leased lines to customer sites. Two such switches have a single T3 trunk connecting the two switches, set aside to support T1s that run just between those two CO switches. Which of the following answers best describes the number of customer T1s the telco can support with these switches and the trunk? a. 20 b. 24 c. 28 d. No exact number; it depends on how much time the customer T1s sit idle.

Answer: C. A T3 trunk supports 28 T1 channels. The number is specific and exact; after being allocated for a particular T1, with TDM and circuit switching, that channel cannot be used to send other bits.

- A customer ordered a full T1 leased line between two sites. The telco implemented that leased line in its network using the T-carrier technology discussed in this chapter, with DS1 and DS3 lines plus switches that use time-division multiplexing (TDM) logic. Into which of the following general

WAN categories does this service fit? a. Ethernet switching b. Packet switching c. Label switching d. Circuit switching

Answer: D. TDM switches act as circuit switching, together creating an end- to-end path for bits from one customer device to the other, with that capacity set aside and reserved for the customer as if it were a digital circuit. Packet switching refers to logic in which the telco switch interprets the bits sent by the customer, to find an address and make a choice per- packet of where to send the bits, possibly choosing a different location for different packets. Label switching could refer to multiprotocol label switching (MPLS) services, and Ethernet switching could refer to Metro Ethernet services.

- A telco customer orders a leased line between sites A and B, with a requested speed of 1.024 Mbps. The customer plans to use an external CSU/DSU at each site. The telco uses only T-carrier technology and none of the more modern options like SONET, ATM, MPLS, or Metro Ethernet. Which of the following answers is true about the speeds used on this link?

a. The physical line between the telco CO and site A uses a line speed of

1.024 Mbps + 8 Kbps for overhead. b. The physical line between the telco CO and site B uses a line speed of 1.544 Mbps. c. The telco rejects the request for the leased line because it is an unsupported speed. d. The CSU/DSU at site A uses a bit rate of 1.024 Mbps when sending data over the line toward the telco CO.

Answer: B. The telco will use only one of three types of lines between the CO and the customer site: DS0, DS1, or DS3. DS0 would not make sense, because it would not support the requested 1.024-Mbps speed. With a T1, the speed in both directions, between the CO switch and the CSU/DSU at the customer site, would be T1 speed, or 1.544 Mbps. Note that the requested 1.024 Mbps is a multiple of 64 Kbps (16 times 64 Kbps), so it is a valid speed option in the T-carrier system.

- A telco customer orders a leased line between sites A and B, with a requested speed of 512 Kbps. The customer plans to use an external CSU/DSU at each site. The telco uses only T-carrier technology and none of the more modern options like SONET, ATM, MPLS, or Metro Ethernet. Which of the

following answers is true about the speeds used between the router and the CSU/DSU? a. The CSU/DSU controls the router’s sending and receiving speeds using clocking. b. The bit rate over the serial cable is

1.536 Mbps (T1 speed minus 8 Kbps overhead). c. There is no bit rate, because the telco rejects the order because of the speed not matching any of the T-carrier line speeds. d. The CSU/DSU and router are peers with regard to clocking, each sending at the speeds configured on each device, with the receiving device being slaved to the sending device’s clocking.

Answer: A. In a leased line, the CSU/DSU always controls the router’s receive and send rates through clocking by changing electrical signals on clocking pins in the serial cable. The router is not in control of the process, neither for sending nor receiving. The CSU/DSU controls the bit rate to match the speed ordered from the telco, because that is all that will fit over the reserved channels of the T1 physical line to the telco anyway. The telco will support a 512-Kbps leased line, because 512 is an even multiple of 64 Kbps.

- Which of the following answers are true about DS1 framing and channels? (Choose two answers.) a. Each frame has 193 bits. b. The frame holds 24 bits, one per channel, plus overhead. c. The frame groups 8 bits together for each of 24 channels, plus overhead. d. If the customer ordered a leased line that ran at 768 Kbps (about half the T1 line rate) the number of frames per second would be about 4000.

Answer: A, C. The frame has 193 bits, starting with 1 overhead bit called the framing bit. After that, the frame has 8 bits for each of the 24 DS0 channels. A DS1 sends 8000 such frames per second, with 8000 * 193 = 1,544,000, which is where the DS1 1.544 Mbps comes from. Finally, one answer mentions a 768-Kbps leased line (fractional T1), but the telco would still use a physical T1 line from the CO to the customer site, which still runs at 1.544 Mbps, with the same frame, so it will still use the usual 8000 frames per second.

- Which of the following data-link protocols was created to be used on leased lines, with support for multiple Layer 3 protocols by including a Type field that identifies the type of packet inside the data-link frame? a. HDLC b. LAPF c. PPP d. SDLC

Answer: C. Point-to-Point Protocol (PPP) was defined specifically for leased lines in a world of multiprotocol routers, and it includes a 2-byte Type field. SDLC, a proprietary IBM protocol, and High-Level Data Link Control (HDLC) were meant for leased lines (and other purposes), but as older protocols, did not include a Type field. LAPF was built for use on Frame Relay access links and does not have a Type field given their specific use in Frame Relay.

- Which fields that exist in both the HDLC and PPP headers have such relatively small use in point-to-point leased lines today, to the point that the PPP standard actually allows the nodes to simply not bother to include these fields when sending PPP frames? (Choose two answers.) a. Flag b. Address c. Control d. FCS

Answer: B, C. The HDLC and PPP Address field, with only a point-to-point topology, has little meaning, because when one device sends, the intended destination is implied. The Control field had many uses for a large variety of early functions in HDLC that applied to the networking world of the 1970s and 1980s, but today, the field often goes unused. Frames still need a Flag field to signal the beginning of the frame and an FCS field to perform error detection.

- Think about the differences in circuit switching using the T-carrier system (ignoring analog circuit switching) versus packet switching. Which of the following answers are true about packet switching, but not about circuit switching? (Choose two answers.) a. The telco physical links are digital circuits. b. The telco switch can queue the data waiting on the congested outgoing trunk to become available. c. The trunks between switches fit the bits into logical time channels on the trunk. d. The switches look at the bits to find an address, and use that address to make a choice of where to send the bits.

Answer: B, D. Packet switches look at the incoming digital electrical signals and interpret them as messages, with a header and an address. The packet switch makes a forwarding decision based on the address in the header. The switch can also add packets to a queue, waiting on the outgoing port to be available for use. Both circuit switches and packet

switches use digital circuits. Only circuit switches fit the bits into logical time channels on the trunk.

- In a Frame Relay network, which of the following terms refers to the customer router that connects to the Frame Relay network? a. DCE b. POP

c. DTE d. Access link

Answer: C. The router connected to the Frame Relay network is the DTE, and the Frame Relay switch is the DCE. The DCE sits in a building called the POP. The physical link between the DCE and DTE is called the access link.

- Which of the following answers is not true about a Frame Relay PVC?

a. Identified by the LAPF header DLCI field b. Defined by the telco ahead of time, when the customer orders the service c. Must exist between two routers before the routers can send Frame Relay frames directly to each other d. Triggers the telco to create a telco leased circuit to support each corresponding PVC

Answer: D. The customer orders Frame Relay by providing many facts, including the various sites, as well as which sites must have a PVC between them. The Frame Relay service provider creates the PVCs at that time, and it assigns a DLCI to identify each PVC. The router pairs without a PVC between them cannot send frames directly to each other. Frame Relay PVCs do not require or allow the creation of a matching physical circuit in the telco network.

- A new Frame Relay customer is considering two competing Frame Relay designs for his WAN. One design uses a full-mesh topology of PVCs between the 20 routers. The second design uses a partial mesh that looks like a hub-and-spoke design. Assuming that all other technical details not mentioned in this question are the same when comparing the two designs, which of these answers are true about the partial-mesh design, but not true about the full-mesh design? (Choose two answers.) a. Not all routers can send a Frame Relay frame directly to each other. b. Frame Relay frames will be forwarded by the network based on the DLCI field in the Frame Relay header. c. The price should be lower. d. Users at spoke sites will not be able to send emails to each other.

Answer: A, C. The partial mesh means that all spoke routers can send IP packets to the hub site, but not directly to each other. However, IP can let them forward IP packets to the hub site and then back out to another spoke site. So IP packets can pass over the Frame Relay network twice to get between spoke sites. The partial-mesh design will use only 20 PVCs, compared to the 190 in the full-mesh design, which with all other details equal, should cost much less.

- Which two of the following WAN services make good use of protocols that enterprises have used for many years, with those protocols being expanded and enhanced to create new types of WAN services? (Choose two answers.) a. ATM b. MPLS c. Metro Ethernet d. PPP

Answer: B, C. MPLS uses IP, routers, and IP routing protocols, along with newly added features, to create WAN services. Metro Ethernet uses Ethernet standards and LAN switches to create WAN services that pass a virtual LAN (VLAN) from one site to another over a WAN, using Ethernet as the Layer 1 access link. ATM can be used to create a WAN service, but it did not exist in the enterprise for long periods before growing to become a WAN service. PPP is a WAN data-link protocol that was first used in WANs.