# ITT Tech

# NT 1210

# Chapter 6

1)An enterprise campus LAN uses wired UTP Ethernet and wireless

802.11 LANs at the edge of the LAN. Looking at a particular floor in a particular building, all the devices connect to either a single Ethernet switch or single wireless access point, both of which sit in a wiring closet in the middle of the space. Which of the following is true about the distance limitations in this LAN? a. All user devices, wired and wireless, can be up to 100 meters away, but no farther. b. All Ethernet user devices can be up to 100 meters away (but no farther), while WLAN devices can typically be farther away than 100 meters. c. All Ethernet user devices can be up to 100

meters away (but no farther), while WLAN devices must typically be closer than 100 meters. d. The Ethernet distances must be decreased to match the maximum distance supported by the wireless LAN.

Answer: C. Ethernet UTP standards all support a 100-meter maximum cable length. The 802.11 wireless LAN standards have a coverage area that is typically less than 100 meters. The literal distance depends on conditions in each wireless LAN. The two types of LANs should have little or no impact on the distances allowed by each.

- IEEE 802.11 standards define a maximum speed for an individual transmission, with some variables that affect 802.11n speeds. Assuming that 802.11n uses the narrower 20-MHz channel and a single stream, which of the following answers matches a particular standard to its maximum bit rate? a. 802.11g: 54 Mbps b. 802.11b: 54 Mbps c. 802.11a: 72 Mbps d. 802.11n: 150 Mbps

Answer: C. The maximums are 802.11a: 54 Mbps; 802.11b: 11 Mbps; 802.11g: 54 Mbps; and 802.11n: 72 Mbps, with the specific options mentioned in the question.

- An enterprise campus LAN uses wired UTP Ethernet and wireless

802.11 LANs at the edge of the LAN. A network engineer uses a WLAN testing tool to view the frames moving through the WLAN, and he notes the speeds of the various frames in a single WLAN that uses a single access point. Which of the following answers list a condition that could have affected the speeds that the engineer observed when testing the WLAN? (Choose two answers.) a. None. The speed must be the same for every frame transmission with the same WLAN and same AP. b. The current radio noise. c. Whether the AP uses CSMA/CA. d. The distance between the client device and the AP.

Answer: B, D. WLAN speeds can vary from frame to frame, and often do, based on conditions. The conditions include the distance between the devices, which impacts the signal strength. The amount of noise also impacts the speed, because it might cause a transmission to be lost, which causes a device to send the next time at a slower speed. The AP must always use CSMA/CA to share the wireless channel.

- An enterprise campus WLAN uses a single 802.11n AP, with 20 clients that all use 802.11n. The AP uses only a single stream, but it does use a double- wide channel, with a maximum data rate for a single transmission of 150 Mbps. The same floor has an Ethernet LAN, with 20 PCs connected using 10BASE-T, UTP cabling. The Ethernet switch and AP both connect to the rest of the network using a Gigabit Ethernet interface. Which of the following statements are true when comparing the wired and wireless LANs on this floor? (Choose two answers.) a. The wired LAN has a larger capacity. b. The wireless LAN has a larger capacity. c. The wired and wireless LAN capacity cannot be compared. d. A single transmission from the AP to a WLAN user device goes faster than the transmission from the Ethernet switch to a wired user device.

Answer: B, D. The bit rate, or transmission rate, is the speed at which the bits are transmitted over the medium. For 802.11n, a 40-MHz bonded channel with one stream, the maximum rate is 150 Mbps. The wired devices use 10BASE-T, which is 10 Mbps, so the WLAN transmissions can be much faster. However, the capacity in a wired LAN that uses full duplex is the sum of all the link speeds, and with 20 links at 10 Mbps, the total capacity on the LAN edge is 200 Mbps. In the WLAN, with a single AP, only a single wireless channel can be used, so the capacity sits at the same 150 Mbps total. So the wired LAN has slightly more capacity.

- A network engineer is designing a campus LAN, both wired and wireless. When planning the wireless LAN, the engineer looks at a scale drawing of the space for each floor and draws the coverage area associated with each AP. Which of the following answers describes the most accurate method the engineer can use to determine the coverage area? a. Do a wireless site survey. b. Read the IEEE standard for the specific 802.11 standard the engineer plans to use for each AP, and find the coverage area maximum distances. c. Read the coverage area guaranteed distances from the AP vendor’s documentation. d. None of the answers is correct.

Answer: A. Because the coverage area depends on many factors, some within the control of the network engineer and some not, the coverage area does not have a set size. The best way to predict the size is to perform a site survey.

- Many groups have a positive impact on the development of wireless LAN standards. Which of the following plays the biggest role in testing and certifying products to prove that they work with other products that have already been certified? a. ITU b. IEEE c. Wi-Fi Alliance d. The vendor that made the new product

Answer: C. The Wi-Fi Alliance, a vendor association, works to make the wireless LAN industry successful, working on behalf of its over 300 members. One if its many roles is to receive new products from vendors before the vendor sells the products, run the products through a series of interoperability tests, and certify that the new product works with other products.

- Which of the following answers define a wireless LAN physical layer standard that was originally ratified by the IEEE before the year 2000? (Choose two answers.) a. 802.11a b. 802.11b c. 802.11c d. 802.11g

Answer: A, B. 802.11a and 802.11b were both formally ratified in 1999, with 802.11g ratified in 2003. 802.11c is not a physical layer standard.

- Which of the following locations is the least likely place to find a wireless LAN? a. In a coffee shop to provide Internet access as a WLAN hotspot b. As a link between two Ethernet switches on two different floors of the same building c. In someone’s home d. At the edge of a campus LAN (where the user devices sit)

Answer: B. Links between Ethernet switches typically need to run faster than links on the edges of the wired LAN. They also benefit from full-duplex operation. Finally, two switches in the same building would likely be somewhat easily connected using a cable to create an Ethernet link. The other three answers all list typical uses for wireless LANs.

- Which of the following wireless LAN topologies use at least one wireless access point? (Choose two answers.) a. BSS b. ESS c. IBSS d. SSID

Answer: A, B. The Basic Service Set (BSS) and Extended Service Set (ESS) both use APs. The Independent BSS (IBSS) does not. The SSID is not a wireless LAN topology.

- Which of the following is true of an ESS WLAN topology but not a BSS WLAN topology? a. Clients sends frames to/from the access point and not directly to/from each other. b. The wireless LAN uses only IEEE 802.11g and more recent standards. c. The wireless LAN has at least two access points. d. The wireless LAN has a minimum coverage area of 300 feet.

Answer: C. A Basic Service Set (BSS), by definition, has a single AP, while an Extended Service Set (ESS) by definition has at least two APs. Neither requires a particular 802.11 standard or a particular coverage area.

- An IT worker buys an AP at the local office supply store and installs it. He then gets out a tape measure and measures the distance from the AP to the cubicles used by Amy (20 feet), Bob (50 feet), and Chuck (150 feet). Each user uses his or her tablet computer from a cubicle. The WLAN uses the 802.11g standard, with a 54-Mbps maximum speed. Which answer is most likely to be true about how these user’s tablets work? a. Chuck’s tablet will either work at 54 Mbps or not work at all. b. Chuck’s and Bob’s tablets never use 54 Mbps to send frames because they sit more than 65 feet from the AP (802.11g’s maximum distance for maximum speed). c. Chuck’s tablet is more likely to work at a slower speed than Alice’s tablet. d. All three tablets will have no trouble sending at 54 Mbps because they sit within the 802.11 maximum distance of 100 meters (330 feet).

Answer: C. All 802.11 wireless LAN standards use a concept in which the standard defines a maximum bit rate, but it also defines a set of alternate slower speeds. The WLAN devices send at a slower speed when the node thinks problems have occurred, for example, when they lose frames because of noise and when the distance from the AP is too far, which makes it harder to interpret the faster radio signals. Chuck’s tablet, which sits far away from the AP, has a greater chance of receiving weaker signals and of having to transmit through more sources of radio noise.

- An IT worker installs a wireless AP, but then realizes through testing that the coverage area does not cover the entire floor of the building. The building is in the shape of a long, skinny rectangle. Which of the following could be used to help change the shape of the coverage area so that it reaches all parts of that floor? (Choose two answers.) a. Increase gain. b.

Replace the antenna with an omnidirectional antenna. c. Replace the antenna with a directional antenna. d. Lower the value of the SSID.

Answer: A, C. The coverage area of an access point can be affected by many factors, but the two discussed in this chapter are the signal gain (the power) and the type of antenna. A directional antenna could work to help cover a space that is not circular, while an omnidirectional antenna makes a circular coverage area. Increasing the transmit power, or gain, increases the size of the coverage area.

- In the IEEE 802.11g standard, how many nonoverlapping frequency channels exist? a. 1 b. 2 c. 3 d. 4

Answer: C. 802.11g uses the 2.4-GHz ISM frequency band, with 11 channels in the United States. Of those, three are nonoverlapping.

- Which of the following types of energy are not part of the electromagnetic spectrum? (Choose two answers.) a. Thermal b. Gamma radiation c. Visible light d. High-pitched sounds

Answer: A, D. The electromagnetic spectrum includes energy such as radio waves, microwaves, visible light, X-rays, and gamma rays. Thermal energy is heat energy, which is not a type of electromagnetic energy. Sound waves are also not a type of electromagnetic energy.

- Which of the following are true about the frequency bands used by WLANs, but not true about the frequency channels used by wireless LANs? (Choose two answers.) a. Defines a set of consecutive frequencies b. Regulated by the FCC in the United States c. Defined by IEEE standards d. Applies to devices that have nothing to do with wireless LANs

Answer: B, D. The frequency band is defined by a regulatory agency, like the FCC in the United States. These frequency bands can be used by a variety of devices, including those that have nothing to do with wireless LANs. The IEEE standards define frequency channels, which are subsets of a frequency band, which WLAN nodes use to transmit data. Both frequency bands and frequency channels define sets of consecutive frequencies.

- Which of the following fields in an 802.11 frame have the same format and meaning as a field in an 802.3 frame? (Choose three answers.) a. Destination MAC Address b. Sender MAC Address c. Preamble d. Frame Check Sequence

Answer: A, B, D. The 802.11 frame uses the same concept of listing the destination device’s MAC address, the sender’s MAC address (both as 6- byte fields), along with a 4-byte FCS field in the trailer. The 802.11 frame does not have a field called a Preamble.

- A user brings his tablet computer into the office where a wireless LAN exists. When the user turns on the tablet, the tablet detects two wireless LANs. Which of the following 802.11 frames or processes plays a key role in the tablet discovering the presence of these wireless LANs? a. Autonegotiation (process) b. Beacon (frame) c. Discover (frame) d. Associate (frame)

Answer: B. For a client to dynamically discover the presence of a wireless LAN, the APs announce their presence by sending a beacon frame. The beacon announces the existence of that particular AP and the wireless LAN.

- A user brings his tablet computer into the office where a wireless LAN exists. The client discovers three nearby APs that make up the Certskills wireless LAN. The client connects to the Certskills WLAN successfully and starts surfing the web. Assuming that the APs are named AP1, AP2, and AP3, which of the following is true about how the client uses this WLAN? a. The client sends data only through the AP from which it received the first beacon. b. The client sends data only through the AP with which it exchanged Associate frames. c. The client sends a frame, and all three APs receive the frame and negotiate as to which one forwards the frame. d. The client load-balances frames by sending one frame to AP1, the next to AP2, then AP3, repeating the process.

Answer: B. In an ESS, each wireless client associates with a single AP for a time. During that time, the client sends and receives all frames to/from that AP. If the client roams, it can reassociate with a better AP, and then at that time, send/receive frames only with that AP. The client does not

consider the order in which it received any beacon frames, and it does not load-balance traffic over multiple APs.

- A wireless client has started using a WLAN, sending all its frames through an AP named AP1. The client sends an 802.11 data frame to the AP. The frame’s destination MAC address is in the same wireless LAN. Which of the following will the AP do when processing this data frame received from the wireless client? (Choose two answers.) a. The AP will discard the frame, expecting that the destination client will have overheard the frame anyway. b. The AP will translate the 802.11 header into an 802.3 header. c. The AP will look for the frame’s destination MAC address in its MAC address table. d. The AP can queue the frame temporarily, waiting for the AP’s turn to send data over the WLAN.

Answer: C, D. In both a BSS and ESS, in which the WLAN uses an AP, all frames flow from client to AP or vice versa, even frames sent from one wireless client to another. So, the AP must process this frame. However, because it does not go onto an Ethernet interface, the AP does not need to translate the header. It will need to make a forwarding decision, using its MAC address table. And, because the AP will forward the frame back out its WLAN port, which uses CSMA/CA, the AP can queue the frame waiting on the AP’s next turn to send data.

- A wireless client has started using a WLAN, sending all its frames through an AP named AP1. The client sends an 802.11 data frame to the AP. The frame’s destination MAC address is on the Ethernet LAN that sits on the other side of the AP. Which of the following will the AP do when processing this data frame received from the wireless client? (Choose two answers.) a. The AP will forward the frame to a router, because routers work well at connecting different types of LANs and WANs. b. The AP will translate the 802.11 header into an 802.3 header. c. The AP will look for the frame’s destination MAC address in its MAC address table. d. The AP can queue the frame temporarily, waiting for the AP’s turn to send data over the WLAN.

Answer: B, C. In both a BSS and ESS, in which the WLAN uses an AP, all frames flow from client to AP or vice versa. For frames flowing from the wireless side to the Ethernet side, the AP can forward the frame because

the 802.11 and 802.3 frames have such similar header and trailer fields. The AP just needs to translate the header in this case from 802.11 to 802.3. The forwarding decision works like an Ethernet switch, with the AP using its MAC address table. The two incorrect answers both have some truth in them, but are not completely true. Routers do excel at connecting unlike types of networks, but an AP can connect 802.11 and 802.3 LANs, translating the headers, without relying on a router. The AP can queue the frame before sending it out onto the Ethernet, but because the AP will not send this frame back onto the WLAN, the AP will not be waiting on the AP’s turn to transmit data over the WLAN.