# ITT Tech

# NT 1210

# Chapter 5

# 1)Which of the following types of LANs was a competitor of Ethernet LANs from the earliest days of wired LANs in the 1980s and into the 1990s? a. ATM b. FDDI c. Token Ring d. Metro Ethernet

Answer: C. The IEEE began the standardization process for Ethernet LANs and Token Ring LANs in 1980, with Ethernet and Token Ring products competing in the 1980s and into the 1990s. FDDI and ATM came later. Metro Ethernet refers to a type of Ethernet standard used for WAN services.

- Which of the following types of LANs is the strongest competitive option to be used instead of Ethernet LANs today? a. ATM b. FDDI c. Token Ring

d. 802.11 LANs

Answer: D. Token Ring, FDDI, and ATM, all of which use cables, have faded from competition as LAN technologies. Only wireless LANs, defined in the IEEE 802.11 standards, offer a strong alternative to Ethernet LANs today.

- Which of the following speeds are not defined as a speed by some Ethernet LAN standard? a. 1 Gbps b. 4 Gbps c. 10 Gbps d. 40 Gbps

Answer: B. Ethernet LAN standards support speeds of 10 Mbps, 100 Mbps, 1 Gbps, 10 Gbps, 40 Gbps, and 100 Gbps.

- Think back to the information listed for Gigabit Ethernet for maximum cabling distances. Which of the answers matches the correct standard with the cabling type and maximum cable length? a. 1000BASE-LX, MM fiber, 550 meters b. 1000BASE-T, UTP, 55 meters c. 1000BASE-SX, MM, 5000 meters d. 1000BASE-T, UTP, 550 meters

Answer: A. The UTP standard for Gigabit, 1000BASE-T, supports 100- meter UTP cable distances. The MM fiber standard, 1000BASE-LX, supports 550-meter MM cable distances. Finally, the SM fiber standard, 1000BASE-SX, supports 5000-meter (5 km) cable distances, with an upgraded standard to support 10 km.

- A campus LAN drawing shows hundreds of user devices cabled to LAN switches on various floors of a building. The per-floor edge switches have a cable connected to other switches that sit on the first floor of the building. All the links from the edge switches to the end-user devices use 100BASE-

T. A PC on floor 2 sends an Ethernet frame to a PC on floor 3, with the frame passing through the switches on the first floor as part of the journey. Which of the following statements is true about the links between the switches in this Ethernet LAN? a. The links must all use 100BASE-T. b. The links could use any Ethernet LAN standard. c. The links could use any UTP-based Ethernet LAN standard. d. The links could use any Ethernet LAN standard from 1 Gbps speeds and faster.

Answer: B. In an Ethernet LAN, the Ethernet frames can flow over Ethernet links defined by any of the Ethernet physical layer standards. The Ethernet frames, by design, are supported by all physical layer standards, just like automobiles can travel over many kinds of roads.

- Which of the following shortcut names defines an IEEE standard that runs at 10,000,000,000 bits per second, and includes some fiber cabling options? a. 10GBASE-T b. 10000BASE-X c. 1000BASE-X d. 10BASE-X e. None of the answers is correct.

Answer: E. The correct answer is 10GBASE-X, with the 10G meaning 10 Gbps and the -X referring to one or more standards that typically include

fiber-optic cabling standards. 10GBASE-T defines the same speed for UTP cabling. 10000BASE-X follows the format for a 10-Gbps standard, but the 10-Gbps speed is only shown as 10G, not as 10000, in the shorthand names. 1000BASE-X is a 1-Gbps standard, and 10BASE-X is a 10-Mbps standard.

- Which of the following is the formal IEEE standard for 1-Gbps Ethernet using UTP cabling? a. 802.3ab b. 802.3ae c. 802.3u d. 802.3z

Answer: A. IEEE standard 802.3ab, named for the IEEE working group’s project number 802.3ab, defines the UTP standard for 1-Gbps Ethernet. 802.3ae defines 10GBASE-X, 10-Gbps Ethernet using fiber optics. 802.3u defines Fast Ethernet, both the fiber and UTP standards. 802.3z defines 1000BASE-X, the fiber-based 1-Gbps standard.

- Imagine that you looked back at the history of prices for Ethernet NICs. You looked at typical prices for 100BASE-T NICs over time. Which of the following answers, according to this chapter, had the strongest impact to lower the prices of the 100BASE-T NICs? (Choose two answers.) a. Overall downward trend for computer technology prices over time b. The introduction of 1000BASE-X products into the market c. The introduction of 1000BASE-T products into the market d. A decrease in prices of fiber-optic cabling

Answer: A, C. This chapter mentioned two items in particular that drive down the prices of Ethernet NICs: the trend of computer technology prices to go down over time and the introduction of a faster-speed Ethernet that uses the same cabling type. The incorrect answers for this question might have had some impact on the prices as well.

- PC A has a 10/100 NIC that supports autonegotiation, and PC B has a 10-Mbps NIC that does not support autonegotiation. Both PCs connect to a LAN switch using a UTP cable. Both switch ports are 10/100/1000 ports that also support IEEE autonegotiation. Which of the following are true about autonegotiation in this small network? (Choose two answers.) a. Autonegotiation works on the switch–to–PC A link, causing PC A to run at 100 Mbps. b. Autonegotiation fails to work on the switch–to–PC B link, causing the switch to run at 1 Gbps. c. Autonegotiation fails to work on the switch–to–PC B link, causing the switch to run at 10 Mbps. d.

Autonegotiation works on the switch–to–PC A link, causing PC A to run at 10 Mbps.

Answer: A, C. When both devices on the ends of a UTP link use autonegotiation, the process works, so they both choose to use the fastest possible speed. In this case, PC A and the switch both use autonegotiation, and their fastest common speed is 100 Mbps. On the PC B link, because PC B does not attempt autonegotiation, the autonegotiation process fails. The switch chooses to use its slowest possible speed, hoping to at least work with the device on the other end of the link, so the switch attempts to run at 10 Mbps.

- PC A and PC B both have 10/100 NICs that support autonegotiation, and both connect to a LAN switch’s ports G1 and G2 using a UTP cable. Both switch ports are 10/100/1000 ports. Both PC NICs and all the switch ports support autonegotiation, but the user of PC B has chosen to both disable autonegotiation and to only use full duplex by making some configuration choices on the PC. Which of the following are true regarding the different device’s choices of whether to use half duplex or full duplex? (Choose two answers.) a. PC A uses full duplex as the result of the autonegotiation process working. b. The switch port G1 uses half duplex as a result of the autonegotiation process working. c. Switch port G2 uses half duplex because autonegotiation fails, making the switch use the default duplex setting. d. Switch port G2 uses full duplex because autonegotiation works and detects that PC B only supports full duplex.

Answer: A, C. On the links from PC A to switch port G1, both devices use autonegotiation, so the process should work. The link supports full duplex because neither node is a hub, and the UTP cable has a transmission path for each direction for which no physical collisions occur. Both claim to support both full and half duplex, and they choose the better option, full duplex. For the PC B link, PC B does not attempt autonegotiation, so switch port G2 must use the default setting for duplex, which is half duplex.

- Think about the various Ethernet standards for a given general category of cabling: UTP, multimode (MM) fiber, and single-mode (SM) fiber. Comparing the standards for one type of cable, for various new standards of various speeds over the years, think about the maximum cabling

distances. Which of the following best describes the most consistent maximum cabling length across a cabling category? a. SM fiber typically supports a maximum cable length of 5 km across all SM standards at all speeds. b. MM fiber typically supports a maximum cable length of 550 meters across all MM standards at all speeds. c. UTP typically supports a maximum cable length of 100 meters across all UTP standards at all speeds. d. All maximum cable distances vary from speed to speed, even for a single category of cable.

Answer: C. The IEEE has consistently (and purposefully) worked to make the UTP standards support a maximum UTP cable length of 100 meters across the range of Ethernet speeds. The maximum cable lengths vary for the various MM and SM standards.

- A network engineer is planning the cabling for a new Ethernet LAN at a new office building. She needs to create an engineering diagram as to which links use straight-through cable pinouts and which use crossover cable pinouts. Which of the following combinations should be marked for crossover pinouts? (Choose two answers.) a. A UTP link from a PC to a switch b. A UTP link from an edge switch to a distribution switch c. A UTP link between two distribution switches d. A fiber link between two distribution switches

Answer: B, C. Links need crossover pinouts for any case in which the nodes on the link use the same pin pair to send data. These links include links with two switches, no matter their roles in the campus LAN design. Fiber-optic cabling does not use the concept of cabling pinouts because they do not have any wires to be placed into pin positions in the connector on the end of a cable.

- PC A sits at a branch office Ethernet LAN. The LAN includes a router, which in turn connects through a WAN to the rest of a large enterprise network. The user at PC A opens a web browser and types in a URL, which eventually causes PC A to send an Ethernet frame. This frame arrives at the router in that same branch office as part of its journey into the rest of the enterprise network. Which of the following statements is true about this Ethernet frame as it passes over the branch office Ethernet LAN? a. The Data field in the frame holds user data only. b. The FCS field sits at the end

of the Ethernet header. c. The Type field identifies the data as an IP packet.

d. The Preamble field lists the length of the frame in bytes.

Answer: C. The Type field identifies the type of data that follows the Ethernet header, and from the context of the question, the user has taken actions that result in the IP protocol building the packet. The Data field lists the user data, as well as headers from other protocol layers. The FCS field sits in the trailer, not the header. The Preamble field is always alternating binary 1 and 0 and does not list the length of the frame.

- PC A sits at a branch office Ethernet LAN. The LAN includes a router, which in turn connects through a WAN to the rest of a large enterprise network. The user at PC A opens a web browser and types in a URL, which eventually causes PC A to send an Ethernet frame. This frame arrives at the router in that same branch office as part of its journey into the rest of the enterprise network. Which of the following statements are true about this Ethernet frame as it passes over the branch office Ethernet LAN? (Choose two answers.) a. Any nodes in the branch LAN that receive this frame use the FCS field to help detect errors. b. The switches in the branch LAN use the source MAC address to help them choose where to forward this one frame. c. The Preamble and SFD fields both come before the Destination Address field in the header. d. The trailer lists the Type and FCS fields.

Answer: A, C. The FCS plays a role in the error-detection process, storing the results of a math formula as calculated by the sender of the frame. The receiver compares that received FCS value with the receiver’s same calculation against the same frame as part of the error-detection process. The header includes, in order, the Preamble, SFD, Destination Address, Source Address, and Type field. The trailer includes only the FCS field.

- Which of the following is true about IEEE Ethernet MAC addresses? a. 48 bits in length b. Can be written in DDN format c. Typically written as 6 hex digits d. Address field(s) in the Ethernet header use a total of 12 bytes

Answer: A. Ethernet defines Media Access Control (MAC) addresses as 48- bit addresses that can be written as the hexadecimal equivalent number of 12 digits, with each digit representing 4 bits. The Ethernet header lists both a destination and source MAC address, 6 bytes each, for a total of 12 bytes

for the two address fields. The MAC address can be written with periods, but it is not dotted-decimal notation (DDN), which lists decimal numbers separated by periods.

- Which of the following entities play a role in helping ensure that universal MAC addresses are unique among all universal MAC addresses in the universe? (Choose two answers.) a. The IEEE b. Ethernet vendors c. Customers that implement Ethernet products d. None of the answers is correct.

Answer: A, B. The IEEE defines the standards and processes by which universal MAC addresses should be assigned. The vendors assign these MAC addresses to products when they are manufactured. The customer that buys the products receives products that already use universally unique MAC addresses.

- In a universal MAC address, which part of the MAC address uses a unique code that identifies the vendor that made the Ethernet product? a. The vendor-assigned code b. The first 2 bytes of the MAC address c. The last 3 bytes of the MAC address d. The OUI

Answer: D. The first half (first 3 bytes) of the universal MAC address identifies the vendor, using an Organizationally Unique Identifier (OUI) assigned to that vendor by the IEEE. The vendor then chooses the value in the second half of the MAC address (3 bytes), called the vendor-assigned code.

- Switch SW1 has four ports: G1, G2, G3, and G4. A frame arrives in port G3. The frame has a destination MAC address of 0000.1234.5678, and the MAC address table lists port G4 as being associated with that MAC address. What action does SW1 take? a. Floods the frame out ports G1, G2, and G4 b. Floods the frame out ports G1, G2, G3, and G4 c. Forwards the frame out port G4 only d. Ignores the frame

Answer: C. Switches forward frames with destination MAC addresses known in their MAC address tables, called known unicast frames, by simply forwarding the frame out the port listed for that entry in the MAC address table.

- Switch SW1 has four ports: G1, G2, G3, and G4. A frame arrives in port G3. The frame has a destination MAC address of FFFF.FFFF.FFFF. What action does SW1 take? a. Floods the frame out ports G1, G2, and G4 b. Floods the frame out ports G1, G2, G3, and G4 c. Forwards the frame out port G4 only d. Ignores the frame

Answer: A. The frame’s destination MAC address is the Ethernet broadcast address, making this frame a broadcast frame. Switches flood broadcast frames out all ports except the port in which the frame arrived.

- Switch SW1 has four ports: G1, G2, G3, and G4. A frame arrives in port G3. The frame has a source MAC address of 0000.3333.3333 and destination MAC address of 0000.1234.5678, and the MAC address table lists port G4 as being associated with that MAC address. Which of the following most likely happened before this frame arrived at SW1? a. SW1 received a frame sent to MAC address 0000.3333.3333 in port G3. b. SW1 received a frame sent to MAC address 0000.1234.5678 in port G4. c. SW1 has received no frames from any devices yet. d. SW1 received a frame sent to MAC address 0000.1234.5678 in port G3.

Answer: B. The only information in the question that lists any information about what happened before the new frame arrived has to do with the current MAC address table. That table has (at least) one entry: an entry for 0000.1234.5678, associated with port G4. For SW1 to have dynamically learned that entry, SW1 must have received a frame that had a source MAC address of 0000.1234.5678, and that frame must have arrived on SW1’s G4 port.