CDA 4213 001L/CIS 6930 012

Fall 2021

CMOS VLSI Design

Question 1 (5 pts): NMOS Transistor

I used 2sk2654 N-Channel MOSFET for this Demonstration

Include the following

- Transistor level diagram

b) Screenshot of schematic

c) Waveform results

d) Explain how you demonstrate that NMOS transistor is good (poor) conductor of 0 (1).

Using the Multimeter range: Diode Test

N-Channel MOSFET

Step 1: Set Range to Diode Test

(+) Test Probe to “source”

(-) Test Probe to “drain”

Reading:

0.3V – 0.97 = Good

OV= Defective

OPEN = Defective

Step 2

(+) Test Probe to “drain”

(-) Test Probe to “source”

Reading:

No Reading = Good

OV = Defective

Step 3: Set Range to Resistance

(+) Test Probe to “source”

(-) Test Probe to “drain”

(+) Test Probe to “drain”

(-) Test Probe to “source

Reading:

High Resistance = Good

Low resistance/Shorted= Defective

Step 4:

(+) Test Probe to “gate”

(-) Test Probe to “source”

(+) Test Probe to “source”

(-) Test Probe to “gate”

Reading:

High Resistance = Good

Low resistance/Shorted= Defective

Step 5

(+) Test Probe to “gate”

(-) Test Probe to “drain”

(+) Test Probe to “drain”

(-) Test Probe to “gate”

Reading:

High Resistance = Good

Low resistance/Shorted= Defective

Question 2. (5 pts.) PMOS Transistor

- Transistor level diagram

b) Screenshot of schematic

c. ) Waveform results

d) Explain how you demonstrate that PMOS transistor is good (poor) conductor of 1 (0).

Using the Multimeter range: Diode Test

P-Channel MOSFET

Step 1: Set Range to Diode Test

(+) Test Probe to “drain”

(-) Test Probe to “source”

Reading:

0.3V – 0.97 = Good

OV= Defective

OPEN = Defective

Step 2

(+) Test Probe to “source”

(-) Test Probe to “drain”

Reading:

No Reading = Good

OV = Defective

Step 3: Set Range to Resistance

(+) Test Probe to “source”

(-) Test Probe to “drain”

(+) Test Probe to “drain”

(-) Test Probe to “source

Reading:

High Resistance = Good

Low resistance/Shorted= Defective

Step 4:

(+) Test Probe to “gate”

(-) Test Probe to “source”

(+) Test Probe to “source”

(-) Test Probe to “gate”

Reading:

High Resistance = Good

Low resistance/Shorted= Defective

Step 5

(+) Test Probe to “gate”

(-) Test Probe to “drain”

(+) Test Probe to “drain”

(-) Test Probe to “gate”

Reading:

High Resistance = Good

Low resistance/Shorted= Defective

Question 3 (10 pts):  Transmission Gate: Using Motor Control Circuit

Include the following

- Transistor level diagram

- Screenshot of schematic

c) Waveform results

Question 4 (10 pts):  2-input Multiplexor using Motor Design

Include the following

- Transistor level diagram

- Screenshot of schematic

- Waveform results