Problem 1: Binary Distillation Mass Balance (Overall + Component)

Problem Statement

A distillation column separates a binary mixture of benzene (B) and toluene (T). A feed stream of 100 kmol/h enters the column with mole fraction xF,B = 0.45 benzene. The distillate product flow rate is 40 kmol/h and has xD,B = 0.95 benzene. Assuming no losses and steady-state operation, determine:
1) Bottoms flow rate (kmol/h)
2) Benzene mole fraction in the bottoms stream (xB,B)
Also compute benzene recovery (%) in the distillate.

Solution (Step-by-Step)

Step 1: Define streams

Feed: F = 100 kmol/h, xF,B = 0.45

Distillate: D = 40 kmol/h, xD,B = 0.95

Bottoms: B = ? kmol/h, xB,B = ?

Step 2: Overall mass balance

F = D + B

100 = 40 + B

B = 60 kmol/h

Step 3: Component (benzene) balance

F xF,B = D xD,B + B xB,B

100(0.45) = 40(0.95) + 60 xB,B

45 = 38 + 60 xB,B

45 − 38 = 60 xB,B

7 = 60 xB,B

xB,B = 7/60 = 0.1167

Step 4: Benzene recovery in distillate

Benzene in feed = 100×0.45 = 45 kmol/h

Benzene in distillate = 40×0.95 = 38 kmol/h

Recovery = (38/45)×100% = 84.44%

Final Answer

Bottoms flow rate B = 60 kmol/h.
Bottoms benzene fraction xB,B ≈ 0.1167.
Benzene recovery in distillate ≈ 84.4%.

Problem 2: Reactor with Single-Pass Conversion + Recycle

Problem Statement

A gas-phase reactor converts A → B with a single-pass conversion of 25% for component A. Fresh feed contains 80 mol% A and 20 mol% inert I, at a total fresh feed rate of 200 kmol/h. The reactor effluent passes through a separator that removes all product B, while unreacted A and inert I are recycled back and mixed with the fresh feed.
Assume:
• Separator removes 100% of B
• No purge stream
• Steady state
Determine:
1) The recycle flow rate (kmol/h)
2) The reactor inlet flow rate (kmol/h)
3) The overall conversion of A based on fresh feed.

Solution (Step-by-Step)

Step 1: Fresh feed composition

Fresh feed total: F0 = 200 kmol/h

A0 = 0.80×200 = 160 kmol/h

I0 = 0.20×200 = 40 kmol/h

Step 2: Let reactor inlet A_R and I_R be the flow rates entering the reactor.

Single-pass conversion Xsp = 0.25

Moles of A reacted = 0.25 A_R

Unreacted A leaving reactor = A_R − 0.25A_R = 0.75A_R

Inert is unchanged: I_out = I_R

Product formed B = 0.25A_R

Step 3: Separator removes all B

Recycle contains only A and I:

Recycle A = 0.75A_R

Recycle I = I_R

Recycle total R = 0.75A_R + I_R

Step 4: Mixing relationship at reactor inlet

A_R = A0 + Recycle A = 160 + 0.75A_R

I_R = I0 + Recycle I = 40 + I_R

Step 5: Solve for A_R

A_R − 0.75A_R = 160

0.25A_R = 160

A_R = 640 kmol/h

Step 6: Inert equation check

I_R = 40 + I_R gives 0 = 40 (impossible)

This indicates no purge causes inert accumulation with recycle.

Engineering conclusion:

A steady-state solution without purge is not physically possible when inert is present and fully recycled.

A purge stream is required.

Step 7: Add a purge fraction p (recommended fix)

If a purge fraction p of recycle is removed, inert can reach steady state.

A full design would specify p and re-solve balances.

Final Answer

No steady-state solution exists with inert present and NO purge.
To achieve steady state, a purge stream must be included to prevent inert accumulation.
(Industry-standard correction: add purge fraction and redo recycle balance.)

Problem 3: Drying Process with Moisture Content on Wet and Dry Basis

Problem Statement

A wet solid containing 30 wt% moisture (water) on a wet basis is dried to 5 wt% moisture on a wet basis. If 1000 lb of wet solid is fed to the dryer, calculate:
1) Mass of dry solid (lb)
2) Initial water in feed (lb)
3) Final water in product (lb)
4) Water removed (lb)
Assume no loss of dry solid during drying.

Solution (Step-by-Step)

Step 1: Interpret moisture content on wet basis

Wet basis moisture fraction = mass water / total mass

Step 2: Initial feed

Total feed = 1000 lb

Initial water = 0.30×1000 = 300 lb

Dry solid = 1000 − 300 = 700 lb

Step 3: Product moisture is 5% wet basis

Let final product total mass = Mp

Final water = 0.05 Mp

Final dry solid = 0.95 Mp

But dry solid is conserved = 700 lb

So:

0.95 Mp = 700

Mp = 700/0.95 = 736.84 lb

Final water = 0.05×736.84 = 36.84 lb

Step 4: Water removed

Water removed = initial water − final water

= 300 − 36.84 = 263.16 lb

Final Answer

Dry solid = 700 lb.
Initial water = 300 lb.
Final product mass ≈ 736.84 lb with water ≈ 36.84 lb.
Water removed ≈ 263.16 lb.