Problem 1: CSTR Design for First-Order Reaction

Problem Statement

A liquid-phase reaction A → Products follows first-order kinetics: −rA = k CA. A CSTR is used with volumetric flow rate v0 = 2.0 m³/min and inlet concentration CA0 = 1.5 kmol/m³. The rate constant is k = 0.8 min⁻¹. Determine the reactor volume required to achieve 70% conversion.

Solution (Step-by-Step)

Step 1: Conversion relationship for CSTR

For a CSTR:

V = v0 (CA0 − CA) / (−rA)

For first order:

−rA = k CA

Step 2: Express CA in terms of conversion X

CA = CA0(1 − X)

Given X = 0.70:

CA = 1.5(1 − 0.70) = 1.5×0.30 = 0.45 kmol/m³

Step 3: Compute rate at exit (same as reactor concentration for CSTR)

−rA = k CA = 0.8×0.45 = 0.36 kmol/m³-min

Step 4: Compute volume

CA0 − CA = 1.5 − 0.45 = 1.05 kmol/m³

V = v0 (CA0 − CA)/(k CA)

V = 2.0×1.05 / 0.36

V = 2.10 / 0.36 = 5.83 m³

Final Answer

Required CSTR volume V ≈ 5.83 m³ for 70% conversion.

Problem 2: PFR Design for Second-Order Reaction

Problem Statement

A liquid-phase irreversible reaction 2A → Products follows rate law: −rA = k CA². A PFR processes a feed with volumetric flow rate v0 = 0.5 m³/min and CA0 = 2.0 kmol/m³. The rate constant is k = 0.25 m³/(kmol·min). Determine reactor volume needed for 80% conversion.

Solution (Step-by-Step)

Step 1: PFR design equation

V = ∫(F_A0 dX)/(−rA)

For constant density liquid:

F_A0 = v0 CA0

CA = CA0(1 − X)

−rA = k CA² = k[CA0(1−X)]²

Step 2: Compute F_A0

F_A0 = 0.5×2.0 = 1.0 kmol/min

Step 3: Setup integral

V = ∫0→0.8 (F_A0 dX)/(k CA0² (1−X)²)

V = F_A0/(k CA0²) ∫0→0.8 dX/(1−X)²

Step 4: Evaluate constants

k CA0² = 0.25×(2.0²)=0.25×4=1.0

So F_A0/(k CA0²)=1.0/1.0 = 1.0

Step 5: Evaluate integral

∫ dX/(1−X)² = [1/(1−X)] from 0 to 0.8

= 1/(0.2) − 1/(1)

= 5 − 1 = 4

Therefore V = 1.0×4 = 4.0 m³

Final Answer

Required PFR volume V ≈ 4.0 m³ for 80% conversion.

Problem 3: Arrhenius Temperature Effect on Rate Constant

Problem Statement

A reaction has Arrhenius parameters: activation energy Ea = 65 kJ/mol. At T1 = 350 K, the rate constant is k1 = 0.12 min⁻¹. Estimate the rate constant at T2 = 400 K. Use R = 8.314 J/mol-K.

Solution (Step-by-Step)

Step 1: Arrhenius ratio form

ln(k2/k1) = −Ea/R (1/T2 − 1/T1)

Convert Ea to J/mol:

Ea = 65,000 J/mol

Step 2: Compute temperature terms

1/T2 − 1/T1 = 1/400 − 1/350

= 0.002500 − 0.002857

= −0.000357 K⁻¹

Step 3: Compute ln ratio

−Ea/R (1/T2 − 1/T1) = −(65000/8.314)(−0.000357)

65000/8.314 = 7817

Multiply: 7817×0.000357 = 2.79

So ln(k2/k1) = 2.79

Step 4: Solve for k2

k2 = k1 e^(2.79) = 0.12×16.3 = 1.96 min⁻¹

Final Answer

Rate constant at 400 K: k2 ≈ 1.96 min⁻¹.