Problem 1: Gas Absorption (Overall Mass Transfer, NTU Method)

Problem Statement

A packed tower is used to absorb a pollutant gas A from air using a liquid solvent. Gas enters with y_in = 0.020 mol fraction A and leaves with y_out = 0.004. The equilibrium relationship is y* = m x with m = 1.5, and the inlet liquid is essentially solute-free (x_in ≈ 0). Assume overall gas-phase mass transfer coefficient is constant and estimate the number of overall gas transfer units (NTU_OG) using the simplified relation:
NTU_OG ≈ ln[(y_in − y*_out)/(y_out − y*_out)]
where y*_out = m x_out, and for lean solvent x_out ≈ y_in/m.
Compute NTU_OG approximately.

Solution (Step-by-Step)

Step 1: Estimate outlet liquid composition for lean solvent

Assume x_out ≈ y_in/m = 0.020/1.5 = 0.01333

Then y*_out = m x_out = 1.5×0.01333 = 0.020

Step 2: Substitute into NTU approximation

NTU_OG ≈ ln[(y_in − y*_out)/(y_out − y*_out)]

But y_in − y*_out = 0.020 − 0.020 = 0

This indicates the simplification is not appropriate in this form.

Engineering correction:

For practical NTU calculations, we use:

NTU_OG = ∫(dy/(y − y*)) with proper operating line.

The simplified expression given collapses because the assumption y*_out = y_in.

Practical result:

A correct NTU requires either operating line data or x_out from liquid flow rate ratio.

Therefore, additional data (L/G) is required to compute NTU realistically.

Final Answer

With the given assumptions, NTU_OG cannot be computed uniquely (missing L/G or x_out).
To complete design, provide solvent flow rate (L) and gas flow rate (G) or outlet liquid composition.

Problem 2: Diffusion Through a Plane Wall (Steady-State)

Problem Statement

Oxygen diffuses through a polymer film of thickness L = 0.5 mm. The oxygen concentration at side 1 is C1 = 2.0×10^−3 mol/m³ and at side 2 is C2 = 0.5×10^−3 mol/m³. The diffusivity of oxygen in the polymer is D = 1.2×10^−10 m²/s. Assuming steady state and 1-D diffusion, calculate the molar flux N (mol/m²-s).

Solution (Step-by-Step)

Step 1: Convert thickness to meters

L = 0.5 mm = 0.0005 m

Step 2: Fick’s first law (steady-state)

N = −D (dC/dx)

For linear profile:

dC/dx = (C2 − C1)/L

= (0.5e−3 − 2.0e−3)/0.0005

= (−1.5e−3)/0.0005

= −3.0 mol/m^4

Step 3: Compute flux

N = −D(−3.0) = 1.2e−10 × 3.0

N = 3.6×10^−10 mol/m²-s

Final Answer

Molar flux N ≈ 3.6×10^−10 mol/m²-s (from side 1 to side 2).

Problem 3: McCabe–Thiele Minimum Reflux Discussion (Conceptual)

Problem Statement

A binary distillation is performed to separate component A from component B. The feed is 40 mol% A, and the distillate target is 95 mol% A with bottoms target 5 mol% A. Explain (qualitatively) how minimum reflux ratio affects the number of stages, and what happens when reflux ratio is increased above minimum.

Solution (Step-by-Step)

Step 1: Minimum reflux ratio concept

At minimum reflux (R_min), the operating lines touch the equilibrium curve at a pinch point.

This causes extremely small driving force for mass transfer in part of the column.

Step 2: Effect on stages

At R = R_min:

• Theoretical stages required → approaches infinity

• Column height and cost become impractical

Step 3: Increasing reflux ratio above minimum

When R > R_min:

• Operating lines move away from equilibrium curve

• Driving force increases

• Required number of stages decreases

Step 4: Practical design trade-off

Higher reflux ratio:

• Lower stages but higher condenser/reboiler duties (energy cost)

Lower reflux ratio:

• Higher stages but lower energy cost

Designers choose an economic optimum, often around 1.2 to 1.5 times R_min depending on economics.

Final Answer

At minimum reflux, stages → infinity due to pinch.
Increasing reflux above minimum reduces stage count but increases energy usage.
Practical design selects R moderately above R_min for cost balance.