Problem 1: Pressure Drop in Pipe (Darcy–Weisbach)

Problem Statement

A process fluid (ρ = 62.4 lbm/ft³) flows through a 2-in schedule pipe of length 120 ft with average velocity 6 ft/s. Assume Darcy friction factor f = 0.02 and pipe diameter D = 2.0 in (0.167 ft). Estimate the pressure drop ΔP in psi using Darcy–Weisbach:
ΔP = f (L/D) (ρ V² / 2)
Use g_c = 32.174 lbm-ft/lbf-s² and convert to psi.

Solution (Step-by-Step)

Step 1: Compute L/D

L/D = 120 / 0.167 = 718.6

Step 2: Dynamic pressure term (in lbf/ft²)

ρV²/2 = 62.4×6²/2 = 62.4×36/2 = 1123.2 lbm/(ft·s²)

Convert using g_c:

(ρV²/2)/g_c = 1123.2/32.174 = 34.9 lbf/ft²

Step 3: Darcy–Weisbach pressure drop

ΔP = f (L/D) (ρV²/2)/g_c

ΔP = 0.02×718.6×34.9 = 501.5 lbf/ft²

Step 4: Convert to psi

1 psi = 144 lbf/ft²

ΔP = 501.5/144 = 3.48 psi

Final Answer

Estimated pressure drop ΔP ≈ 3.5 psi.

Problem 2: Reactor Heat Removal (Cooling Jacket Duty)

Problem Statement

An exothermic reaction in a jacketed batch reactor releases heat at a rate of 90,000 Btu/hr during its peak period. Cooling water enters the jacket at 65°F and exits at 85°F. Assume water cp = 1.0 Btu/lbm-°F. Determine required cooling water flow rate (lbm/hr).

Solution (Step-by-Step)

Step 1: Temperature rise of cooling water

ΔT = 85 − 65 = 20°F

Step 2: Heat absorbed by cooling water

Q = m cp ΔT

m = Q/(cpΔT)

m = 90,000/(1×20) = 4,500 lbm/hr

Final Answer

Cooling water requirement ≈ 4,500 lbm/hr.

Problem 3: Relief Valve Sizing (Conceptual + Mass Balance Basis)

Problem Statement

A storage vessel contains a volatile liquid. During an external fire scenario, vapor generation increases and the vessel pressure rises. Explain what key data is required to size a pressure relief valve (PRV) and outline the general calculation steps used in PRV sizing.

Solution (Step-by-Step)

Step 1: Key data required

• Vessel design pressure and set pressure of PRV

• Fire heat input estimate (API fire case)

• Wetted surface area of vessel

• Latent heat of vaporization (for boiling fluid)

• Fluid properties (molecular weight, compressibility for gas)

• Backpressure conditions and discharge coefficients

Step 2: Estimate vapor generation rate

Fire case typically uses:

Q_fire = C1 × A_wetted^0.82

Vapor mass rate m_dot = Q_fire / ΔHvap

Step 3: Convert vapor rate to required relief area

Depending on whether relieving fluid is gas/vapor or two-phase, use API equations.

For vapor relief:

A = (m_dot × √T × Z) / (C × Kd × P)

Step 4: Select standard valve orifice size

Choose next larger standard orifice.

Verify allowable overpressure and backpressure limits.

Step 5: Documentation and compliance

PRV sizing must follow relevant standards (e.g., API 520/521) and be reviewed as part of PHA/HAZOP documentation.

Final Answer

PRV sizing requires fire heat input, wetted area, vaporization data, fluid properties, and set pressure.
General steps: estimate fire duty → vapor generation → required relief area → select standard valve size per API.