Problem 1: Equivalent Resistance (Series-Parallel Network)

Problem Statement

Find the equivalent resistance between terminals A and B. R1 = 4 Ω is in series with a parallel combination of R2 = 6 Ω and R3 = 3 Ω.

Solution

First find parallel part:

1/Rp = 1/R2 + 1/R3 = 1/6 + 1/3 = 1/6 + 2/6 = 3/6

Rp = 6/3 = 2 Ω

Total resistance:

Req = R1 + Rp = 4 + 2 = 6 Ω

Final Answer

Equivalent resistance Req = 6 Ω.

Problem 2: Thevenin Equivalent (DC Network)

Problem Statement

A 12 V source is in series with R1 = 4 Ω feeding a load terminal. A resistor R2 = 8 Ω is connected from the load terminal to ground. Find Thevenin voltage and Thevenin resistance seen at the load terminal.

Solution

Thevenin voltage is the open-circuit voltage at the terminal:

Voltage divider:

Vth = 12 × (R2/(R1+R2)) = 12×(8/(4+8))

Vth = 12×(8/12) = 8 V

To find Rth, deactivate the source (replace 12 V source by short circuit):

Then R1 and R2 are in parallel:

Rth = (R1R2)/(R1+R2) = (4×8)/(12) = 32/12 = 2.67 Ω

Final Answer

Vth = 8 V and Rth ≈ 2.67 Ω.

Problem 3: RC Charging Circuit (Time Constant)

Problem Statement

An RC circuit has R = 10 kΩ and C = 100 µF connected to a 5 V step input. Find the time constant and the capacitor voltage at t = 1 s.

Solution

Time constant:

τ = RC = 10,000 × 100×10^−6 = 1 s

Charging equation:

Vc(t) = V(1 − e^(−t/τ))

At t = 1 s:

Vc = 5(1 − e^(−1))

e^(−1) ≈ 0.3679

Vc ≈ 5(1 − 0.3679) = 5×0.6321 = 3.16 V

Final Answer

τ = 1 s and Vc(1 s) ≈ 3.16 V.