Problem 1: Impedance of Series RLC

Problem Statement

A series circuit has R = 10 Ω, L = 50 mH, C = 100 µF operating at f = 60 Hz. Find the impedance magnitude |Z|.

Solution

Angular frequency:

ω = 2πf = 2π×60 = 377 rad/s

Inductive reactance:

XL = ωL = 377×0.05 = 18.85 Ω

Capacitive reactance:

XC = 1/(ωC) = 1/(377×100×10^−6)=1/0.0377=26.53 Ω

Net reactance:

X = XL − XC = 18.85 − 26.53 = −7.68 Ω

Impedance magnitude:

|Z| = √(R^2 + X^2) = √(10^2 + 7.68^2)

= √(100 + 59.0) = √159.0 = 12.61 Ω

Final Answer

|Z| ≈ 12.6 Ω.

Problem 2: Power Factor and Real Power

Problem Statement

A single-phase load draws I = 12 A from a 120 V (rms) source at power factor 0.8 lagging. Find apparent power S, real power P, and reactive power Q.

Solution

Apparent power:

S = VI = 120×12 = 1440 VA

Real power:

P = S×pf = 1440×0.8 = 1152 W

Reactive power:

Q = S sin(acos(pf))

cosφ = 0.8 => sinφ = √(1−0.8^2)=0.6

Q = 1440×0.6 = 864 VAR

Final Answer

S = 1440 VA, P = 1152 W, Q = 864 VAR (lagging).

Problem 3: Three-Phase Power (Balanced Load)

Problem Statement

A balanced 3-phase load is supplied at line voltage VL = 480 V and draws line current IL = 20 A at power factor 0.9 lagging. Find total real power.

Solution

Total real power for balanced 3-phase:

P = √3 VL IL pf

P = 1.732×480×20×0.9

= 1.732×9600×0.9

= 16,627×0.9 ≈ 14,964 W

Final Answer

Total real power ≈ 15.0 kW.