Problem 1: Transfer Function of RLC Circuit

Problem Statement

For a series RLC circuit with output taken across the capacitor, derive the transfer function H(s)=Vc(s)/Vin(s).

Solution

Impedances in s-domain:

Z_R = R, Z_L = sL, Z_C = 1/(sC)

Total series impedance:

Z_total = R + sL + 1/(sC)

Voltage divider across capacitor:

H(s) = Z_C / Z_total

H(s) = (1/(sC)) / (R + sL + 1/(sC))

Multiply numerator and denominator by sC:

H(s) = 1 / (sCR + s^2LC + 1)

Therefore:

H(s) = 1 / (LC s^2 + RC s + 1)

Final Answer

H(s) = 1 / (LC s^2 + RC s + 1).

Problem 2: Steady-State Error (Type-1 System)

Problem Statement

A unity-feedback system has open-loop transfer function G(s)=K/(s(s+5)). Find K so that the steady-state error for a unit ramp input is ess = 0.1.

Solution

For unity feedback, velocity error constant Kv:

Kv = lim(s→0) sG(s) = lim(s→0) s×K/(s(s+5)) = K/5

For a ramp input:

ess = 1/Kv

Given ess = 0.1:

0.1 = 1/(K/5) = 5/K

So K = 50

Final Answer

Required gain K = 50.

Problem 3: Fourier Series Coefficient (Square Wave)

Problem Statement

A 50% duty cycle square wave has amplitude ±1 with period T. Write the Fourier series (only non-zero terms) for this waveform.

Solution

A symmetric square wave with amplitude ±1 has zero DC component and only odd harmonics:

The sine-series form:

x(t) = (4/π)[sin(ωt) + (1/3)sin(3ωt) + (1/5)sin(5ωt) + ...]

where ω = 2π/T

Even harmonics are zero due to half-wave symmetry.

Final Answer

x(t) = (4/π) Σ_{n=1,3,5...} (1/n) sin(nωt), ω=2π/T.