Problem 1: Boolean Simplification (K-map Result)

Problem Statement

Simplify the Boolean expression: F = A'B + AB. Provide the simplified output.

Solution

Factor B:

F = B(A' + A)

Since (A' + A) = 1:

F = B

Final Answer

Simplified expression: F = B.

Problem 2: Nyquist Sampling Rate

Problem Statement

A signal contains frequency components up to 5 kHz. Determine the minimum sampling rate to avoid aliasing.

Solution

Nyquist sampling theorem:

fs ≥ 2 fmax

fs ≥ 2×5 kHz = 10 kHz

Final Answer

Minimum sampling rate = 10 kHz.

Problem 3: Magnetic Flux Density in a Solenoid

Problem Statement

An air-core solenoid has N = 500 turns, length l = 0.25 m, and current I = 2 A. Find magnetic flux density B. Use B = μ0 (N/l) I, where μ0 = 4π×10^−7 H/m.

Solution

Turns per meter:

N/l = 500/0.25 = 2000 turns/m

B = μ0 (N/l) I = (4π×10^−7)×2000×2

= 4π×10^−7×4000

= 16,000π×10^−7

≈ 5.03×10^−3 T

Final Answer

B ≈ 5.0×10^−3 Tesla (≈ 5 mT).