Isolated Spread Footing Design for Axial Column Load

Problem
Design a square isolated spread footing for a centrally loaded concrete column.

Given
• Service dead + live column load, P = 180 kips
• Column size = 16 in × 16 in
• Allowable soil bearing pressure, qa = 3.0 ksf
• Concrete strength, f'c = 4,000 psi
• Steel yield strength, fy = 60,000 psi

Solution
Step 1: Footing area required
Areq = P / qa = 180 / 3.0 = 60 ft²

Select footing size:
Adopt 8 ft × 8 ft = 64 ft² (OK)

Step 2: Actual bearing pressure
q = P / A = 180 / 64 = 2.81 ksf (≤ 3.0 ksf → OK)

Step 3: Footing thickness (one-way shear control, preliminary)
Adopt thickness t = 20 in (common practical starting point)
Effective depth d ≈ 17 in

Step 4: Flexure steel (strip method)
Cantilever projection each side:
p = (8 − 1.33)/2 ≈ 3.33 ft
Moment at face (per ft width):
Mu ≈ q p² / 2

Mu ≈ 2.81 × 3.33² / 2 ≈ 15.6 kip-ft/ft

Provide bottom reinforcement both directions:
• #6 bars @ 8 in o.c. each way (bottom)

Final Footing Design (Summary)
• Footing size: 8 ft × 8 ft
• Thickness: 20 in
• Reinforcement: #6 @ 8 in o.c. each way (bottom)
• Concrete cover: 3 in (cast against soil)