Problem 1: Ideal Otto Cycle Thermal Efficiency

Problem Statement

An ideal Otto cycle has a compression ratio r = 9. The ratio of specific heats is k = 1.4. Determine the air-standard thermal efficiency.

Solution

For an ideal Otto cycle:

η_th = 1 − 1/(r^(k−1))

Compute r^(k−1):

r^(k−1) = 9^(0.4)

9^(0.4) ≈ exp(0.4 ln 9) = exp(0.879) ≈ 2.41

Therefore:

η_th = 1 − 1/2.41 = 1 − 0.415 ≈ 0.585

Final Answer

Thermal efficiency ≈ 0.585 (≈ 58.5%).

Problem 2: Rankine Cycle (Turbine Specific Work)

Problem Statement

A simple Rankine cycle operates between 3000 psia (boiler) and 1 psia (condenser). Steam enters the turbine at 3000 psia and 1000°F and exits isentropically to 1 psia. Estimate turbine specific work output using typical steam-table values: h1 ≈ 3580 kJ/kg, s1 ≈ 6.8 kJ/kg-K; at 1 psia, h_f ≈ 191 kJ/kg, s_f ≈ 0.65 kJ/kg-K, h_fg ≈ 2392 kJ/kg, s_fg ≈ 7.5 kJ/kg-K.

Solution

Isentropic expansion implies s2 = s1.

At 1 psia, quality x:

s2 = s_f + x s_fg

6.8 = 0.65 + x(7.5)

6.15 = 7.5x

x ≈ 0.82

Enthalpy at exit:

h2 = h_f + x h_fg

h2 = 191 + 0.82(2392)

0.82×2392 ≈ 1961

h2 ≈ 2152 kJ/kg

Turbine specific work:

w_t = h1 − h2 = 3580 − 2152 = 1428 kJ/kg

Final Answer

Turbine specific work output ≈ 1428 kJ/kg.

Problem 3: Heat Exchanger Effectiveness (Counterflow)

Problem Statement

In a counterflow heat exchanger, hot water enters at 180°F and leaves at 120°F with mass flow rate 2 lbm/s. Cold water enters at 70°F with mass flow rate 3 lbm/s. Take cp = 1.0 Btu/lbm-°F for both streams. Find effectiveness ε.

Solution

Heat transfer rate:

Q = m_h cp (T_h,in − T_h,out)

Q = 2×1×(180−120) = 120 Btu/s

Capacity rates:

C_h = 2 Btu/s-°F, C_c = 3 Btu/s-°F

C_min = 2 Btu/s-°F

Maximum possible heat transfer:

Q_max = C_min (T_h,in − T_c,in) = 2×(180−70)=220 Btu/s

Effectiveness:

ε = Q/Q_max = 120/220 = 0.545

Final Answer

Effectiveness ε ≈ 0.545 (≈ 54.5%).