Problem 1: Simply Supported Beam (Maximum Bending Moment)

Problem Statement

A simply supported beam of span L = 12 ft carries a uniform load w = 500 lb/ft over the full span. Determine the maximum bending moment.

Solution

For a simply supported beam with full-span UDL:

M_max = wL^2/8

M_max = 500×12^2/8

12^2 = 144

M_max = 500×144/8 = 500×18 = 9000 lb-ft

Final Answer

Maximum bending moment = 9000 lb-ft.

Problem 2: Axial Stress and Elongation

Problem Statement

A steel rod (E = 29×10^6 psi) has length L = 40 in and diameter d = 1.0 in. It carries an axial tensile load P = 20,000 lbf. Find axial stress and elongation.

Solution

Area A = (π/4)d^2 = 0.785 in^2

Stress:

σ = P/A = 20000/0.785 ≈ 25,478 psi ≈ 25.5 ksi

Elongation:

δ = PL/(AE)

δ = (20000×40)/(0.785×29×10^6)

δ ≈ 0.035 in

Final Answer

σ ≈ 25.5 ksi and δ ≈ 0.035 in.

Problem 3: Torsion in Solid Circular Shaft

Problem Statement

A solid circular shaft has diameter d = 2.0 in and transmits torque T = 1500 lb-ft. Find maximum shear stress using τ_max = Tc/J, J = πd^4/32.

Solution

Convert torque:

T = 1500×12 = 18,000 lb-in

Radius c = 1.0 in

J = π(2^4)/32 = 1.571 in^4

τ_max = Tc/J = 18000×1/1.571 ≈ 11,458 psi ≈ 11.46 ksi

Final Answer

Maximum shear stress τ_max ≈ 11.46 ksi.