Problem 1: Pipe Flow Reynolds Number & Friction Factor (Blasius)

Problem Statement

Water at 68°F flows through a smooth 2.0-in diameter pipe at velocity V = 8 ft/s. Take ν = 1.1×10^−5 ft²/s. Compute Reynolds number and estimate Darcy friction factor f = 0.3164 Re^(−0.25).

Solution

D = 2.0 in = 0.167 ft

Re = VD/ν = 8×0.167/(1.1×10^−5)

Re ≈ 1.21×10^5

Re^(0.25) ≈ 18.6

f ≈ 0.3164/18.6 = 0.0170

Final Answer

Re ≈ 1.21×10^5 and f ≈ 0.017.

Problem 2: Forced Convection Flat Plate (Average Nusselt Number)

Problem Statement

Air at 1 atm and 77°F flows over a flat plate length L = 1.5 m at U = 10 m/s. Take ν = 1.5×10^−5 m²/s and Pr = 0.71. Estimate average Nu using Nu_L = 0.664 Re_L^(1/2) Pr^(1/3).

Solution

Re_L = UL/ν = 10×1.5/(1.5×10^−5)=1.0×10^6

Re_L^(1/2)=1000

Pr^(1/3)=0.71^(1/3)≈0.892

Nu_L = 0.664×1000×0.892 ≈ 592

Final Answer

Average Nusselt number Nu_L ≈ 592.

Problem 3: Radiation Heat Exchange Between Large Plates

Problem Statement

Two large parallel plates: T1 = 800 K, ε1 = 0.7; T2 = 500 K, ε2 = 0.5. Find net radiation heat flux. Use σ = 5.67×10^−8 W/m²-K^4.

Solution

q = σ(T1^4−T2^4) / ( (1/ε1)+(1/ε2)−1 )

Denominator = 1/0.7 + 1/0.5 − 1 = 2.4286

T1^4 = 800^4 = 4.096×10^11

T2^4 = 500^4 = 6.25×10^10

Difference = 3.471×10^11

Numerator = 5.67e−8×3.471e11 ≈ 19,680 W/m²

q ≈ 19,680/2.4286 = 8,105 W/m²

Final Answer

q ≈ 8.1×10^3 W/m² (from Plate 1 to Plate 2).