Problem 1: Turning Operation (Cutting Speed to RPM)

Problem Statement

A turning operation is performed on a workpiece diameter D = 2.5 in. Recommended cutting speed is V = 300 ft/min. Find spindle speed N (rpm). Use V = (πDN)/12.

Solution

N = 12V/(πD)

N = 12×300/(π×2.5)

N ≈ 458 rpm

Final Answer

Spindle speed ≈ 460 rpm.

Problem 2: Double Fillet Weld (Effective Throat Area)

Problem Statement

A double fillet weld of length 4 in is used on both sides of a plate. Weld leg size is 1/4 in. Find total effective throat area. Use throat t = 0.707×leg.

Solution

t = 0.707×0.25 = 0.1768 in

Area per weld line A1 = t×L = 0.1768×4 = 0.707 in²

Total (two lines): Atotal = 2×0.707 = 1.414 in²

Final Answer

Total effective throat area ≈ 1.41 in².

Problem 3: Helical Compression Spring (Spring Rate)

Problem Statement

A helical spring has wire diameter d = 0.25 in, mean coil diameter D = 2.0 in, active coils Na = 10, and shear modulus G = 11.5×10^6 psi. Find spring rate k using k = G d^4/(8 D^3 Na).

Solution

d^4 = 0.25^4 = 0.00390625

D^3 = 2^3 = 8

k = (11.5×10^6×0.00390625)/(8×8×10)

Numerator = 44,921.9

Denominator = 640

k ≈ 70.2 lb/in

Final Answer

Spring rate k ≈ 70 lb/in.