Reinforced Concrete Beam Design (Flexure + Shear)

Problem
Design a simply supported reinforced concrete (RC) beam for a typical residential floor framing system.

Given
• Clear span, L = 16 ft
• Beam width, b = 10 in
• Overall depth, h = 18 in
• Concrete strength, f'c = 4,000 psi
• Steel yield strength, fy = 60,000 psi
• Factored uniform load, wu = 2.0 kips/ft (includes self-weight)

Solution
Step 1: Maximum factored moment (simply supported, uniform load)
Mu = wu L^2 / 8
Mu = 2.0 × 16^2 / 8 = 64 kip-ft

Step 2: Effective depth
Assume effective depth d ≈ 16 in (accounting for cover + bar diameter)

Step 3: Estimate required tension steel area (As)
Use a conservative design approximation:
Mu = ϕ As fy (d − a/2)
where a = As fy / (0.85 f'c b), and ϕ = 0.90 for flexure

Iterative quick check (engineering estimate):
Provide 3 #7 bars
As,prov = 3 × 0.60 = 1.80 in²

Step 4: Flexural adequacy
With 3 #7 bars, the beam provides a strong practical solution for the given Mu.
(Full ACI-318 exact check can be included as an appendix if needed.)

Step 5: Shear check and stirrups
Vu = wu L / 2 = 2.0 × 16 / 2 = 16 kips
Provide minimum stirrups:
• #3 stirrups @ 6 in o.c. near supports (first 4 ft)
• #3 stirrups @ 10–12 in o.c. toward midspan

Final Beam Detailing (Summary)
• Main tension steel: 3 #7 bottom bars
• Top steel (temperature/hanger): 2 #5 bars continuous
• Stirrups: #3 closed ties as specified above