Retaining Wall Stability (Sliding + Overturning)

Problem
Check stability of a gravity retaining wall supporting granular backfill.

Given
• Retained height, H = 12 ft
• Unit weight of soil, γ = 120 pcf
• Soil friction angle, ϕ = 30°
• Wall unit weight, γc = 150 pcf
• Wall base width, B = 7.5 ft (provided)
• Coefficient of friction at base, μ = 0.50

Solution
Step 1: Active earth pressure coefficient
Ka = (1 − sinϕ) / (1 + sinϕ)
Ka = (1 − 0.5) / (1 + 0.5) = 0.333

Step 2: Total active thrust
Pa = 0.5 Ka γ H²
Pa = 0.5 × 0.333 × 120 × 12²
Pa ≈ 2,880 lb/ft = 2.88 kips/ft

Line of action at H/3 = 4 ft above base.

Step 3: Overturning moment
Mo = Pa × (H/3) = 2.88 × 4 = 11.52 kip-ft/ft

Step 4: Wall weight (approx.)
Assume wall cross-section area Aw ≈ B × H / 2 = 7.5 × 12 / 2 = 45 ft²
W = γc Aw = 150 × 45 = 6,750 lb/ft = 6.75 kips/ft

Resisting moment about toe (approx. lever arm ~ B/3 = 2.5 ft)
Mr ≈ 6.75 × 2.5 = 16.88 kip-ft/ft

Factor of safety against overturning:
FSOT = Mr / Mo = 16.88 / 11.52 = 1.47
Typical target is 2.0 → improvement recommended.

Step 5: Sliding check
Resisting friction = μW = 0.50 × 6.75 = 3.38 kips/ft
FSS = μW / Pa = 3.38 / 2.88 = 1.17
Typical target is 1.5 → improvement recommended.

Recommended Fixes
• Increase base width (B) or add a shear key to improve sliding resistance
• Increase wall weight (thicker stem/base) to improve overturning resistance
• Use granular drainage + reduce hydrostatic pressure behind wall

Final Recommendation
Provided section is marginal; revise B to ~9–10 ft or include a shear key for stability compliance.