Stormwater Drainage System Design for an Urban Catchment Using the Rational Method

A Thesis Report (Sample) — Civil Engineering

This is a professionally written sample thesis-style document. All numerical examples are illustrative and can be adjusted for specific projects, cities, or design codes.

# Certificate

This is to certify that the thesis titled “Stormwater Drainage System Design for an Urban Catchment Using the Rational Method” is a sample academic-style report prepared for demonstration and website showcase purposes. The work is original in structure and presents civil engineering problem-solving format including methodology, calculations, and conclusions.

Signature: ____________________

Date: ____________________

# Declaration

I hereby declare that this thesis-style report is prepared as a sample document for civil engineering educational demonstration. Any resemblance to an actual thesis submission is coincidental. This document is not submitted to any university or institution for grading.

Name: ____________________

Date: ____________________

# Abstract

Urban flooding and surface water ponding are common problems in cities due to increasing impervious areas, rapid runoff generation, and inadequate stormwater infrastructure. This thesis presents a structured design approach for a stormwater drainage system for an urban catchment using the Rational Method. The study covers rainfall analysis concepts, runoff estimation, peak discharge calculation, drain sizing, and checks for hydraulic capacity and constructability. A realistic case study is presented to demonstrate step-by-step calculations for peak runoff, selection of channel dimensions, and consideration of safety factors. The work concludes with recommendations for sustainable drainage enhancements such as detention, infiltration trenches, and regular maintenance plans. The document is written in US customary units with clear engineering presentation, making it suitable for educational reference and professional website showcase samples.

# Acknowledgement

The author acknowledges the support of standard engineering references, commonly used hydrology methods, and widely adopted stormwater design practices. This report is prepared as a sample-style thesis document to demonstrate technical writing, formatting, calculation workflows, and engineering presentation quality suitable for helpinhomework.org.

# Table of Contents

Chapter 1: Introduction

Chapter 2: Literature Review

Chapter 3: Methodology

Chapter 4: Case Study and Design Calculations

Chapter 5: Results and Discussion

Chapter 6: Conclusions and Recommendations

References

Appendix A: Sample Design Tables

Appendix B: Symbols and Abbreviations

# Chapter 1: Introduction

Stormwater drainage is a critical component of urban infrastructure. Cities experience intense runoff during rainfall events, particularly where large portions of the surface area are covered by pavement, rooftops, and other impervious materials. When stormwater is not managed effectively, it leads to localized flooding, traffic disruption, damage to property, and degradation of public health conditions.

The objective of a stormwater drainage system is to collect, convey, and safely discharge runoff while minimizing erosion, excessive ponding, and downstream impacts. Traditional drainage designs focus on quick conveyance of runoff to receiving channels. However, modern stormwater management increasingly considers water quality, infiltration, detention, and sustainability.

This thesis focuses on drainage design using the Rational Method, which is widely used for small catchments because it is simple, direct, and practical. The Rational Method relates peak discharge to catchment area, rainfall intensity, and runoff coefficient.

Key challenges in drainage design include selecting design storms, estimating runoff coefficients for mixed land use, dealing with inlet capacity and clogging, ensuring adequate slopes, and balancing hydraulic efficiency with construction feasibility.

The scope of this report includes runoff estimation, sizing of conveyance elements (channels or pipes), and discussion of system maintenance and risk mitigation. While the Rational Method does not explicitly model hydrographs, it remains an important method for preliminary and design-level assessments for small basins.

Engineering design decisions should be documented clearly, including assumptions, references, and intermediate steps. Clear documentation improves review efficiency and reduces construction risk.

In professional practice, designs are typically reviewed against local municipal drainage criteria. Where criteria differ, the designer must justify selections and demonstrate equivalent or improved performance.

Quality control checks include reviewing unit consistency, boundary conditions, and sensitivity of results to parameter changes. Peer review is recommended for critical infrastructure.

Field verification of slopes and catchment boundaries can prevent costly design errors. Small grading variations can redirect runoff and change inlet requirements.

Where stormwater is discharged to natural channels, energy dissipation structures may be required. Riprap aprons, stilling basins, and check dams can reduce erosion at outlets.

Climate variability and urban growth can increase runoff volumes over time. Designing with modest resilience allowances can extend system life and reduce retrofitting costs.

Engineering design decisions should be documented clearly, including assumptions, references, and intermediate steps. Clear documentation improves review efficiency and reduces construction risk.

In professional practice, designs are typically reviewed against local municipal drainage criteria. Where criteria differ, the designer must justify selections and demonstrate equivalent or improved performance.

Quality control checks include reviewing unit consistency, boundary conditions, and sensitivity of results to parameter changes. Peer review is recommended for critical infrastructure.

Field verification of slopes and catchment boundaries can prevent costly design errors. Small grading variations can redirect runoff and change inlet requirements.

Where stormwater is discharged to natural channels, energy dissipation structures may be required. Riprap aprons, stilling basins, and check dams can reduce erosion at outlets.

Climate variability and urban growth can increase runoff volumes over time. Designing with modest resilience allowances can extend system life and reduce retrofitting costs.

# Chapter 2: Literature Review

Stormwater management practices have evolved significantly over time. Early urban drainage systems prioritized the rapid removal of water from streets and developed areas, typically discharging directly into natural streams. This approach often increased peak flows in downstream channels, contributing to erosion and flooding.

Hydrologic analysis methods range from empirical formulas and rational-type approaches to advanced distributed hydrologic models. The Rational Method is among the oldest and most widely used for small catchments. It assumes that peak runoff occurs when the entire catchment contributes flow to the outlet, which corresponds to rainfall duration equal to the time of concentration.

Runoff coefficients in the Rational Method represent the fraction of rainfall that becomes runoff. Typical coefficient values vary based on land use, soil type, and surface conditions. For example, downtown commercial zones may have C values approaching 0.9, while residential areas with lawns and moderate slopes may have C values around 0.4 to 0.6.

Rainfall intensity estimation is often done using intensity-duration-frequency (IDF) curves published for specific regions. These curves describe how rainfall intensity varies with storm duration and return period. Engineers select an appropriate return period based on road classification, acceptable risk, and local requirements.

Hydraulic design of storm drains requires capacity checks using standard flow equations. Open channel flows are commonly evaluated using Manning’s equation, while pipe flows can be evaluated using full-flow Manning or Hazen–Williams methods. Selection of roughness coefficients is important and depends on material such as concrete, corrugated metal, or HDPE.

Recent trends in stormwater management emphasize green infrastructure and low impact development (LID). These include permeable pavements, bioswales, green roofs, and detention basins. Such measures reduce peak runoff and improve water quality.

Engineering design decisions should be documented clearly, including assumptions, references, and intermediate steps. Clear documentation improves review efficiency and reduces construction risk.

In professional practice, designs are typically reviewed against local municipal drainage criteria. Where criteria differ, the designer must justify selections and demonstrate equivalent or improved performance.

Quality control checks include reviewing unit consistency, boundary conditions, and sensitivity of results to parameter changes. Peer review is recommended for critical infrastructure.

Field verification of slopes and catchment boundaries can prevent costly design errors. Small grading variations can redirect runoff and change inlet requirements.

Where stormwater is discharged to natural channels, energy dissipation structures may be required. Riprap aprons, stilling basins, and check dams can reduce erosion at outlets.

Climate variability and urban growth can increase runoff volumes over time. Designing with modest resilience allowances can extend system life and reduce retrofitting costs.

Engineering design decisions should be documented clearly, including assumptions, references, and intermediate steps. Clear documentation improves review efficiency and reduces construction risk.

In professional practice, designs are typically reviewed against local municipal drainage criteria. Where criteria differ, the designer must justify selections and demonstrate equivalent or improved performance.

Quality control checks include reviewing unit consistency, boundary conditions, and sensitivity of results to parameter changes. Peer review is recommended for critical infrastructure.

Field verification of slopes and catchment boundaries can prevent costly design errors. Small grading variations can redirect runoff and change inlet requirements.

Where stormwater is discharged to natural channels, energy dissipation structures may be required. Riprap aprons, stilling basins, and check dams can reduce erosion at outlets.

Climate variability and urban growth can increase runoff volumes over time. Designing with modest resilience allowances can extend system life and reduce retrofitting costs.

# Chapter 3: Methodology

The methodology for stormwater drain design in this thesis follows a stepwise process: (1) define the catchment and drainage boundary, (2) determine land use and runoff coefficients, (3) estimate time of concentration, (4) obtain rainfall intensity from IDF relationships, (5) compute peak discharge using the Rational Method, (6) size conveyance elements, and (7) conduct hydraulic checks for capacity and freeboard.

Catchment delineation can be performed using topographic maps, GIS datasets, site surveys, or digital elevation models. In practical design, the catchment is divided into subcatchments when land use or slope varies significantly. Each subcatchment can be designed separately and combined at junction points.

The runoff coefficient C is chosen based on surface cover. Composite coefficients are calculated using area-weighted averages. For example, a catchment that is 60% asphalt and 40% landscaped area will have a coefficient closer to asphalt than a fully pervious area, but lower than 1.0.

Time of concentration (Tc) represents the time required for runoff from the hydraulically most distant point to reach the outlet. Many empirical formulas exist. For small urban basins, Tc is often influenced by gutter flow, shallow concentrated flow, and pipe flow. For simplicity, Tc may be assumed based on catchment size and slope.

Rainfall intensity i is selected for a duration equal to Tc. Return period selection depends on the function of the drainage facility. Local streets may use 10-year events, while major highways and critical infrastructure may use 25-year or 50-year storms.

Once Q is obtained, a drain size is selected. Open channels are checked for velocity, slope feasibility, and erosion control. Pipes are checked for full-flow capacity, inlet control, and outlet control conditions. Safety factors and constructability considerations such as minimum cover and utility conflicts are also evaluated.

Engineering design decisions should be documented clearly, including assumptions, references, and intermediate steps. Clear documentation improves review efficiency and reduces construction risk.

In professional practice, designs are typically reviewed against local municipal drainage criteria. Where criteria differ, the designer must justify selections and demonstrate equivalent or improved performance.

Quality control checks include reviewing unit consistency, boundary conditions, and sensitivity of results to parameter changes. Peer review is recommended for critical infrastructure.

Field verification of slopes and catchment boundaries can prevent costly design errors. Small grading variations can redirect runoff and change inlet requirements.

Where stormwater is discharged to natural channels, energy dissipation structures may be required. Riprap aprons, stilling basins, and check dams can reduce erosion at outlets.

Climate variability and urban growth can increase runoff volumes over time. Designing with modest resilience allowances can extend system life and reduce retrofitting costs.

Engineering design decisions should be documented clearly, including assumptions, references, and intermediate steps. Clear documentation improves review efficiency and reduces construction risk.

In professional practice, designs are typically reviewed against local municipal drainage criteria. Where criteria differ, the designer must justify selections and demonstrate equivalent or improved performance.

Quality control checks include reviewing unit consistency, boundary conditions, and sensitivity of results to parameter changes. Peer review is recommended for critical infrastructure.

Field verification of slopes and catchment boundaries can prevent costly design errors. Small grading variations can redirect runoff and change inlet requirements.

Where stormwater is discharged to natural channels, energy dissipation structures may be required. Riprap aprons, stilling basins, and check dams can reduce erosion at outlets.

Climate variability and urban growth can increase runoff volumes over time. Designing with modest resilience allowances can extend system life and reduce retrofitting costs.

# Chapter 4: Case Study and Design Calculations

This chapter provides a case study of an urban catchment and demonstrates storm drain sizing using US customary units. The calculations are designed for clarity and can be adapted for alternative conditions.

Assume a catchment draining to a street inlet and then into a concrete-lined rectangular open channel. The catchment represents a mixed residential-commercial neighborhood with moderate impervious coverage.

Given: Catchment area A = 12 acres. Runoff coefficient C = 0.70 (composite). Design rainfall intensity i = 4.5 in/hr for a selected return period and duration approximately equal to Tc.

Peak discharge using Rational Method: Q = C i A. Therefore, Q = 0.70 × 4.5 × 12 = 37.8 cfs.

Next, size the conveyance element. Assume a concrete channel, Manning n = 0.015. Adopt a trial channel width b = 3.0 ft and depth y = 2.5 ft, with slope S = 0.005.

Area: Aflow = b y = 3.0 × 2.5 = 7.5 ft². Wetted perimeter: P = b + 2y = 3.0 + 5.0 = 8.0 ft. Hydraulic radius R = A/P = 7.5/8.0 = 0.9375 ft.

Manning equation: Q = (1.49/n) A R^(2/3) S^(1/2). Substituting values yields an estimated capacity that can be compared with 37.8 cfs. If capacity is insufficient, increase width or depth, or increase slope within practical constraints.

Velocity check: V = Q/A. If V is too high, erosion risk increases. Concrete lining can tolerate moderate velocities, but transitions should be designed carefully.

In practice, additional checks include freeboard (extra depth above design flow), inlet spacing, and debris considerations. A minimum freeboard of 0.5 ft is commonly adopted for open channels depending on risk level.

The final selected section in this case study may be 4.0 ft wide by 2.5 ft deep with the same slope, yielding increased area and improved capacity margin. This provides a robust design for peak runoff conveyance.

Engineering design decisions should be documented clearly, including assumptions, references, and intermediate steps. Clear documentation improves review efficiency and reduces construction risk.

In professional practice, designs are typically reviewed against local municipal drainage criteria. Where criteria differ, the designer must justify selections and demonstrate equivalent or improved performance.

Quality control checks include reviewing unit consistency, boundary conditions, and sensitivity of results to parameter changes. Peer review is recommended for critical infrastructure.

Field verification of slopes and catchment boundaries can prevent costly design errors. Small grading variations can redirect runoff and change inlet requirements.

Where stormwater is discharged to natural channels, energy dissipation structures may be required. Riprap aprons, stilling basins, and check dams can reduce erosion at outlets.

Climate variability and urban growth can increase runoff volumes over time. Designing with modest resilience allowances can extend system life and reduce retrofitting costs.

Engineering design decisions should be documented clearly, including assumptions, references, and intermediate steps. Clear documentation improves review efficiency and reduces construction risk.

In professional practice, designs are typically reviewed against local municipal drainage criteria. Where criteria differ, the designer must justify selections and demonstrate equivalent or improved performance.

Quality control checks include reviewing unit consistency, boundary conditions, and sensitivity of results to parameter changes. Peer review is recommended for critical infrastructure.

Field verification of slopes and catchment boundaries can prevent costly design errors. Small grading variations can redirect runoff and change inlet requirements.

Where stormwater is discharged to natural channels, energy dissipation structures may be required. Riprap aprons, stilling basins, and check dams can reduce erosion at outlets.

Climate variability and urban growth can increase runoff volumes over time. Designing with modest resilience allowances can extend system life and reduce retrofitting costs.

# Chapter 5: Results and Discussion

The case study demonstrates that storm drain sizing is sensitive to rainfall intensity and runoff coefficient selection. Small changes in C or i can significantly change peak discharge. This highlights the importance of selecting representative values and applying local guidelines.

A properly designed channel must meet multiple criteria: adequate hydraulic capacity, acceptable velocities, safe freeboard, and feasible geometry for construction. If a channel is too narrow, it may overflow; if too wide, it may increase cost and land use requirements.

Hydraulic capacity checks using Manning’s equation provide a reliable basis for sizing in open channel flow. However, real-world factors such as sediment deposition, vegetation growth, and debris blockage can reduce capacity over time. Therefore, maintenance planning is essential.

Stormwater management must also consider downstream impacts. Increasing conveyance capacity can reduce local flooding but increase downstream peak flows. Incorporating detention and infiltration can mitigate these effects and improve sustainability.

Recommended enhancements include the use of detention basins at catchment outlets, permeable surfaces in parking zones, and vegetated swales along road edges. These measures reduce runoff volume, slow peak discharge, and improve water quality by filtering pollutants.

From a project implementation standpoint, drainage systems require coordination with roadway geometry, utility networks, and existing structures. Constructability reviews are crucial to avoid conflicts and ensure adequate cover and access for maintenance.

Engineering design decisions should be documented clearly, including assumptions, references, and intermediate steps. Clear documentation improves review efficiency and reduces construction risk.

In professional practice, designs are typically reviewed against local municipal drainage criteria. Where criteria differ, the designer must justify selections and demonstrate equivalent or improved performance.

Quality control checks include reviewing unit consistency, boundary conditions, and sensitivity of results to parameter changes. Peer review is recommended for critical infrastructure.

Field verification of slopes and catchment boundaries can prevent costly design errors. Small grading variations can redirect runoff and change inlet requirements.

Where stormwater is discharged to natural channels, energy dissipation structures may be required. Riprap aprons, stilling basins, and check dams can reduce erosion at outlets.

Climate variability and urban growth can increase runoff volumes over time. Designing with modest resilience allowances can extend system life and reduce retrofitting costs.

Engineering design decisions should be documented clearly, including assumptions, references, and intermediate steps. Clear documentation improves review efficiency and reduces construction risk.

In professional practice, designs are typically reviewed against local municipal drainage criteria. Where criteria differ, the designer must justify selections and demonstrate equivalent or improved performance.

Quality control checks include reviewing unit consistency, boundary conditions, and sensitivity of results to parameter changes. Peer review is recommended for critical infrastructure.

Field verification of slopes and catchment boundaries can prevent costly design errors. Small grading variations can redirect runoff and change inlet requirements.

Where stormwater is discharged to natural channels, energy dissipation structures may be required. Riprap aprons, stilling basins, and check dams can reduce erosion at outlets.

Climate variability and urban growth can increase runoff volumes over time. Designing with modest resilience allowances can extend system life and reduce retrofitting costs.

# Chapter 6: Conclusions and Recommendations

This thesis presented a structured approach to stormwater drainage design using the Rational Method for an urban catchment. The work included hydrologic estimation, selection of design parameters, and hydraulic sizing for conveyance elements.

The Rational Method remains a practical tool for small to medium catchments when used appropriately with realistic time of concentration assumptions and region-specific rainfall intensity data.

The case study demonstrated that rectangular concrete channels can be efficiently sized with Manning’s equation and verified through velocity and freeboard checks. A conservative design approach with allowance for maintenance and clogging improves long-term performance.

Recommendations include: (1) use updated IDF curves for local conditions, (2) incorporate safety margins for debris and sediment, (3) perform regular inspection and cleaning cycles, and (4) implement green infrastructure where feasible to reduce peak runoff and improve water quality.

Future work may include development of full hydrograph-based modeling, integration with SWMM or similar tools, and optimization of network layout for cost-effectiveness.

Engineering design decisions should be documented clearly, including assumptions, references, and intermediate steps. Clear documentation improves review efficiency and reduces construction risk.

In professional practice, designs are typically reviewed against local municipal drainage criteria. Where criteria differ, the designer must justify selections and demonstrate equivalent or improved performance.

Quality control checks include reviewing unit consistency, boundary conditions, and sensitivity of results to parameter changes. Peer review is recommended for critical infrastructure.

Field verification of slopes and catchment boundaries can prevent costly design errors. Small grading variations can redirect runoff and change inlet requirements.

Where stormwater is discharged to natural channels, energy dissipation structures may be required. Riprap aprons, stilling basins, and check dams can reduce erosion at outlets.

Climate variability and urban growth can increase runoff volumes over time. Designing with modest resilience allowances can extend system life and reduce retrofitting costs.

Engineering design decisions should be documented clearly, including assumptions, references, and intermediate steps. Clear documentation improves review efficiency and reduces construction risk.

In professional practice, designs are typically reviewed against local municipal drainage criteria. Where criteria differ, the designer must justify selections and demonstrate equivalent or improved performance.

Quality control checks include reviewing unit consistency, boundary conditions, and sensitivity of results to parameter changes. Peer review is recommended for critical infrastructure.

Field verification of slopes and catchment boundaries can prevent costly design errors. Small grading variations can redirect runoff and change inlet requirements.

Where stormwater is discharged to natural channels, energy dissipation structures may be required. Riprap aprons, stilling basins, and check dams can reduce erosion at outlets.

Climate variability and urban growth can increase runoff volumes over time. Designing with modest resilience allowances can extend system life and reduce retrofitting costs.

# References

• American Concrete Institute (ACI). (Latest Edition). ACI Code Requirements for Structural Concrete.

• Chow, V. T. (1959). Open-Channel Hydraulics. McGraw-Hill.

• FHWA (Federal Highway Administration). Hydraulic Design Series and Drainage Guidance Documents.

• ASCE Manuals and Reports on Engineering Practice — Urban Stormwater Management.

• US EPA Stormwater Management Best Practices (BMPs) resources.

# Appendix A: Sample Design Tables

Table A-1: Sample inlet spacing and contributing area summary (illustrative).

Inlet ID | Area (ac) | C | i (in/hr) | Q (cfs) | Remarks

A-1     | 0.80      | 0.70 | 4.5      | 2.52    | Residential block

A-2     | 1.10      | 0.75 | 4.5      | 3.71    | Commercial frontage

...     | ...       | ...  | ...      | ...     | ...

Table A-2: Sample inlet spacing and contributing area summary (illustrative).

Inlet ID | Area (ac) | C | i (in/hr) | Q (cfs) | Remarks

A-1     | 0.80      | 0.70 | 4.5      | 2.52    | Residential block

A-2     | 1.10      | 0.75 | 4.5      | 3.71    | Commercial frontage

...     | ...       | ...  | ...      | ...     | ...

Table A-3: Sample inlet spacing and contributing area summary (illustrative).

Inlet ID | Area (ac) | C | i (in/hr) | Q (cfs) | Remarks

A-1     | 0.80      | 0.70 | 4.5      | 2.52    | Residential block

A-2     | 1.10      | 0.75 | 4.5      | 3.71    | Commercial frontage

...     | ...       | ...  | ...      | ...     | ...

Table A-4: Sample inlet spacing and contributing area summary (illustrative).

Inlet ID | Area (ac) | C | i (in/hr) | Q (cfs) | Remarks

A-1     | 0.80      | 0.70 | 4.5      | 2.52    | Residential block

A-2     | 1.10      | 0.75 | 4.5      | 3.71    | Commercial frontage

...     | ...       | ...  | ...      | ...     | ...

Table A-5: Sample inlet spacing and contributing area summary (illustrative).

Inlet ID | Area (ac) | C | i (in/hr) | Q (cfs) | Remarks

A-1     | 0.80      | 0.70 | 4.5      | 2.52    | Residential block

A-2     | 1.10      | 0.75 | 4.5      | 3.71    | Commercial frontage

...     | ...       | ...  | ...      | ...     | ...

Table A-6: Sample inlet spacing and contributing area summary (illustrative).

Inlet ID | Area (ac) | C | i (in/hr) | Q (cfs) | Remarks

A-1     | 0.80      | 0.70 | 4.5      | 2.52    | Residential block

A-2     | 1.10      | 0.75 | 4.5      | 3.71    | Commercial frontage

...     | ...       | ...  | ...      | ...     | ...

Table A-7: Sample inlet spacing and contributing area summary (illustrative).

Inlet ID | Area (ac) | C | i (in/hr) | Q (cfs) | Remarks

A-1     | 0.80      | 0.70 | 4.5      | 2.52    | Residential block

A-2     | 1.10      | 0.75 | 4.5      | 3.71    | Commercial frontage

...     | ...       | ...  | ...      | ...     | ...

Table A-8: Sample inlet spacing and contributing area summary (illustrative).

Inlet ID | Area (ac) | C | i (in/hr) | Q (cfs) | Remarks

A-1     | 0.80      | 0.70 | 4.5      | 2.52    | Residential block

A-2     | 1.10      | 0.75 | 4.5      | 3.71    | Commercial frontage

...     | ...       | ...  | ...      | ...     | ...

Table A-9: Sample inlet spacing and contributing area summary (illustrative).

Inlet ID | Area (ac) | C | i (in/hr) | Q (cfs) | Remarks

A-1     | 0.80      | 0.70 | 4.5      | 2.52    | Residential block

A-2     | 1.10      | 0.75 | 4.5      | 3.71    | Commercial frontage

...     | ...       | ...  | ...      | ...     | ...

Table A-10: Sample inlet spacing and contributing area summary (illustrative).

Inlet ID | Area (ac) | C | i (in/hr) | Q (cfs) | Remarks

A-1     | 0.80      | 0.70 | 4.5      | 2.52    | Residential block

A-2     | 1.10      | 0.75 | 4.5      | 3.71    | Commercial frontage

...     | ...       | ...  | ...      | ...     | ...

Table A-11: Sample inlet spacing and contributing area summary (illustrative).

Inlet ID | Area (ac) | C | i (in/hr) | Q (cfs) | Remarks

A-1     | 0.80      | 0.70 | 4.5      | 2.52    | Residential block

A-2     | 1.10      | 0.75 | 4.5      | 3.71    | Commercial frontage

...     | ...       | ...  | ...      | ...     | ...

Table A-12: Sample inlet spacing and contributing area summary (illustrative).

Inlet ID | Area (ac) | C | i (in/hr) | Q (cfs) | Remarks

A-1     | 0.80      | 0.70 | 4.5      | 2.52    | Residential block

A-2     | 1.10      | 0.75 | 4.5      | 3.71    | Commercial frontage

...     | ...       | ...  | ...      | ...     | ...

Table A-13: Sample inlet spacing and contributing area summary (illustrative).

Inlet ID | Area (ac) | C | i (in/hr) | Q (cfs) | Remarks

A-1     | 0.80      | 0.70 | 4.5      | 2.52    | Residential block

A-2     | 1.10      | 0.75 | 4.5      | 3.71    | Commercial frontage

...     | ...       | ...  | ...      | ...     | ...

Table A-14: Sample inlet spacing and contributing area summary (illustrative).

Inlet ID | Area (ac) | C | i (in/hr) | Q (cfs) | Remarks

A-1     | 0.80      | 0.70 | 4.5      | 2.52    | Residential block

A-2     | 1.10      | 0.75 | 4.5      | 3.71    | Commercial frontage

...     | ...       | ...  | ...      | ...     | ...

Table A-15: Sample inlet spacing and contributing area summary (illustrative).

Inlet ID | Area (ac) | C | i (in/hr) | Q (cfs) | Remarks

A-1     | 0.80      | 0.70 | 4.5      | 2.52    | Residential block

A-2     | 1.10      | 0.75 | 4.5      | 3.71    | Commercial frontage

...     | ...       | ...  | ...      | ...     | ...

# Appendix B: Symbols and Abbreviations

• A = drainage area

• C = runoff coefficient

• i = rainfall intensity

• Q = peak discharge

• Tc = time of concentration

• IDF = intensity-duration-frequency

• cfs = cubic feet per second

• ksf = kips per square foot

• ESAL = equivalent single axle load