Stormwater Runoff & Drain Sizing (Rational Method)

Problem
Estimate peak runoff from a small urban catchment and size a storm drain.

Given
• Drainage area, A = 7 acres
• Runoff coefficient, C = 0.70
• Rainfall intensity, i = 4.0 in/hr
• Target drain type: rectangular concrete channel

Solution
Step 1: Peak discharge (Rational Method)
Q = C i A
Q = 0.70 × 4.0 × 7 = 19.6 cfs

Step 2: Select a practical channel size
Assume allowable velocity V ≈ 4 ft/s (lined channel)

Required flow area:
Aflow = Q / V = 19.6 / 4 = 4.9 ft²

Adopt a rectangular channel:
• Width = 2.0 ft
• Flow depth = 2.5 ft
Area = 2.0 × 2.5 = 5.0 ft² (OK)

Step 3: Check capacity
Qcap = A × V = 5.0 × 4 = 20 cfs (≥ 19.6 cfs)

Final Answer (Summary)
• Peak runoff: 19.6 cfs
• Provide rectangular channel: 2.0 ft wide × 2.5 ft flow depth (concrete lined)